AK-SR7

PRODUCT TEST REPORT

By

AKOM TEST CENTER

August 2003

Part A. SPECIFICATION TEST	3

1	Product Specification	3

2	System Specific Function Test	4

3	BIOS Function Test	7

4	Broacom BCM5703 GbE Basic Function Test	17

Part B. OPERATION SYSTEM ENVIRONMENT TEST	19

1	Microsoft Windows 98 SE Operation System Test	19

2	Microsoft Windows NT Operation System Test	錯誤! 尚未定義書籤。

3	Microsoft Windows 2000 Operation System Test	33

4	Microsoft Windows XP Operation System Test	47

5	Linux Operation System Test	59

Part C. Performance Measurement	61

Copyright	66

Disclaimer	66

Part A. SPECIFICATION TEST

- Product Specification

| Features | Description |
| --- | --- |
| CPU |  Single P4 3.06GHz+ (FSB 400/533) |
| DRAM |  3 DIMMS support up to 3GB DDR 200/266/333 |
| Chipset |  SIS 645DX + SIS 961B |
| LAN |  One Broadcom BCM5703 GbE port |
| IDE Channels |  2 South bridge ATA133 IDE connector  8 HPT372 ATA133 IDE connector |
| Floppy |  One 1,44MB |
| On-board I/O |  2 USB1.1 ports  PS/2 keyboard and mouse connector  2 parallel port (COM port)  1 LPT port |
| Expansion Slot |  Two PCI 2.2 Specification compliance slots |
| Others |  On-board debug LED  On-board power on & reset switch |
| Hardware RAID support |  4 HPT372 C\controllers support RAID-0 RAID-1 RAID-0+1 JBOD up to 4 HDDs |
| BIOS available |  SR7_24 |
| HPT372 BIOS available |  HPT V2.32 |

- System Specific Function Test

| CPU Test |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Vender | Model | L2 Catch | Voltage | Clock Multiplier | Bus Speed | Remark |
| Intel | Pentium 4-1500 | 256KB | 1.75V | 15X | 100 MHz |  |
|  | Pentium 4-1600 | 256KB | 1.75V | 16X | 100 MHz |  |
|  |  | 512KB | 1.45V |  |  |  |
|  | Pentium 4-1700 | 256KB | 1.75V | 17X | 100 MHz |  |
|  |  | 512KB | 1.45V |  |  |  |
|  | Pentium 4-1800 | 256KB | 1.75V | 18X | 100 MHz |  |
|  |  | 512KB | 1.45V |  |  |  |
|  | Pentium 4-1900 | 256KB | 1.45V | 19X | 100 MHz |  |
|  | Pentium 4-2000 | 512KB | 1.45V | 20X | 100 MHz |  |
|  | Pentium 4-2200 | 512KB | 1.45V | 22X | 100 MHz |  |
|  | Pentium 4-2266 | 512KB | 1.45V | 17X | 133 MHz |  |
|  | Pentium 4-2400 | 512KB | 1.45V | 18X | 133 MHz |  |
| Memory Test |  |  |  |  |  |  |
| Type | Size | Vendor | Module |  | Chip Vendor | Chip Component |
| 184 Pin DIMM Module (266MHz) | 128Mbytes | Micron | MT8VDDT1664AG-265A1 |  | Micron | MT46V16M8-75A |
|  |  | Nanya | PC266 128MB |  | Nanya | NT5DS16M8AT-75B |
|  |  | Samsung | 128MB DDR PC266 |  | Samsung | K4H280838B-TCB0 |
|  |  | Twinmos | PC266 128MB |  | Hyundai | HY5DU28822 |
|  |  | Twinmos | PC2100 128MB DDR-2 |  | Hyundai | HY5DU28822T-H |
|  |  | VIA | NT128D64S88A0G-7K |  | Nanya | NT5DS16M8AT-7K |
|  |  | Winbond | W9412FADA-7 |  | Winbond | W942516AH-7 |
|  | 256Mbytes | Kingmax | MPLB62D-68K |  | Kingmax | KDL684T4A2A-06 |
|  |  | Kingmax | MPLB62D-68KX2 |  | Kingmax | KDL684T4A2A-07A |
|  |  | Samsung | PC266 DDR |  | Samsung | K4H280838B-TCB0 |
|  |  | Samsung | RLDT1256 256MB |  | Samsung | K4H280838B-TCB0 |
|  |  | Samsung | M368L3313BT1-CB0 |  | Samsung | K4H280838B-TCB0 |
|  |  | Transcend | PC266 256MB DDR |  | Mosel | V58C2128804SAT75 |
|  |  | Twinmos | PC2100 256MB DDR/CL2 |  | Hynix | HY5DU28822AT-K |
|  |  | Winbond | W9425GARA-7 |  | Winbond | W942525GARA-7 |
|  | 512Mbytes | Infineon | HYS64D64020GU-7-A |  | Infineon | HYB250256800AT-7 |

| Memory Test |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 184 Pin DIMM Module (333 MHz) | 256Mbytes |  | Kingmax | MPMB62D-68KX3 |  |  | Kingmax | KDL884T4A2A-05 |
|  |  |  | Nanya | 256MB DDR333 |  |  | Nanya | NT5DS16M8AT-6 |
|  | 512Mbytes |  | Samsung | M368L6423CTL-CB3 |  |  | Samsung | K4H560838C-TCB3 |
| Front Panel Interface |  |  |  |  |  |  |  |  |
| Type |  | Test Item | O.S. Environment |  | Result | Features Tested |  |  |
| Power Button |  | One Touch | CMOS |  | Pass | See system power working normally |  |  |
|  |  | >4 Sec’s | CMOS |  | Pass | 4 Sec’s shut down since last system hole |  |  |
| Reset Button |  | Reset System | Windows 2K/XP |  | Pass | Reset system works properly |  |  |
| Power LED |  | Normal Mode | Windows 2K/XP |  | Pass | Power LED must not blink |  |  |
|  |  | Suspend Mode | Windows 2K/XP |  | Pass | Power LED must blink |  |  |
| HDD LED |  | Active | Windows 2K/XP |  | Pass | HDD LED must active while IDE HDD access |  |  |
| Sleep LED |  | Normal Mode | Windows 2K/XP |  | Pass | Sleep LED must not blink |  |  |
|  |  | Suspend Mode | Windows 2K/XP |  | Pass | Sleep LED not blink |  |  |
| LAN LED |  | Link | Windows 2K/XP |  | Pass | LAN LED must bright |  |  |
|  |  | Active | Windows 2K/XP |  | pass | LAN LED must blink while network data access |  |  |
| Floppy Drive |  | Active | Windows 2K/XP |  | Pass | Floppy drive should works normally |  |  |
| Back Panel Interface |  |  |  |  |  |  |  |  |
| Type |  | Test Item | O.S. Environment |  | Result | Features Tested |  |  |
| PS/2 Connectors |  | Keyboard | Windows 2K/XP |  | Pass | PS/2 keyboard should work normally |  |  |
|  |  | Mouse | Windows 2K/XP |  | Pass | PS/2 mouse should work normally |  |  |
| Serial Ports |  | COM 1 | Windows 2K/XP |  | Pass | Serial port should work normally |  |  |
|  |  | COM 2 | Windows 2K/XP |  | Pass | Serial port should work normally |  |  |
| Parallel Port |  | Enable | Windows 2K/XP |  | Pass | Parallel port should work normally |  |  |
|  |  | Disable | Windows 2K/XP |  | Pass | Parallel port should not be found by O.S. |  |  |
| LAN Port |  | Enable | Windows 2K/XP |  | Pass | LAN port should work normally |  |  |
|  |  | Disable | Windows 2K/XP |  | Pass | LAN port Should not be found by O.S. |  |  |
| USB Port |  | USB 1 | Windows 2K/XP |  | Pass | USB port should work normally |  |  |
|  |  | USB 2 | Windows 2K/XP |  | Pass | USB port should work normally |  |  |

| On-board Connectors and Slots |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset | Active |  | Windows 2K/XP CMOS |  |  | Pass |  | System could be reboot while press reset switch |
| Power on | Active |  | Windows 2K/XP CMOS |  |  | Pass |  | System could be power on while press power on switch |
| Debug LED | Active |  | CMOS |  |  | Pass |  | Check Debug LED could work normally |
| RTCCLR | Clear CMOS |  | CMOS |  |  | Pass |  | System should reentry BIOS with minimum setting |
| System LED | Active |  | CMOS |  |  | Pass |  | System LED should on after connect power cable |
| System Battery | Active |  | CMOS |  |  | Pass |  | System battery should be available |
| 5V / +5V Power Control | Keyboard |  | Windows 2K/XP |  |  | Pass |  | 5V or +5V standby power should have function |
|  | USB 0-1 |  | Windows 2K/XP |  |  | Pass |  | 5V or +5V standby power should have function |
| On-board Connectors and Slots |  |  |  |  |  |  |  |  |
| Fan Connectors |  | CPU Fan |  | CMOS | Pass |  | Fan speed should be in range, work and with ACPI |  |
|  |  | Power Fan |  | CMOS | Pass |  | Power speed should be in range, work and with ACPI |  |
|  |  | Chassis Fan |  | CMOS | Pass |  | Chassis speed should be in range, work and with ACPI |  |
| System Slots |  | Floppy |  | Windows 2K/XP | Pass |  | Floppy slot should be function and no mechanical problem |  |
|  |  | CPU |  | Windows 2K/XP | Pass |  | CPU slot should be function and no mechanical problem |  |
|  |  | DRAM Slots |  | Windows 2K/XP | Pass |  | DRAM slot should be function and no mechanical problem |  |
|  |  | PCI Slots |  | Windows 2K/XP | Pass |  | PCI slot should be function and no mechanical problem |  |
|  |  | IDE 1 |  | Windows 2K/XP | Pass |  | IDE1 should be function and no mechanical problem |  |
|  |  | IDE 2 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 3 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 4 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 5 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 6 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 7 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 8 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 9 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |
|  |  | IDE 10 |  | Windows 2K/XP | Pass |  | should be function and no mechanical problem |  |

- BIOS Function Test

  - Server Board BIOS Function Test

| SoftMenu Setup |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| CPU Operation Speed | Default (FSB 133*4) | CMOS | Fail | CPU speed should be auto detected by BIOS |
|  | Default (FSB 100 *4) | CMOS | Pass | CPU speed should be auto detected by BIOS |
|  | User define (FSB 100*4) | CMOS | Pass | CPU speed should be set by manual |
|  | User define (FSB 100*4) | CMOS | Pass | CPU speed should be set by manual |
|  | Default Multiplier Factor | CMOS | Pass | CPU multiplier should be auto detected by BIOS |
|  | User define Multiplier Factor | CMOS | Pass | CPU multiplier should be set manual |
| FSB Rate 133*4 DRAM/AGP/PCI | 4/3/2/1 | CMOS | Pass | DRAM/AGP/PCI frequency can be set |
|  | 4/4/2/1 | CMOS | Pass | DRAM/AGP/PCI frequency can be set |
|  | 4/5/2/1 | CMOS | Pass | DRAM/AGP/PCI frequency can be set |
| FSB Rate 100*4 DRAM/AGP/PCI | 48/80/30/15 | CMOS | Pass | DRAM/AGP/PCI frequency can be set |
|  | 3/3/2/1 | CMOS | Pass | DRAM/AGP/PCI frequency can be set |
|  | 3/4/2/1 | CMOS | Pass | DRAM/AGP/PCI frequency can be set |
| CPU Power Supply Core Voltage | Default Willimat (1.75V) | CMOS | Pass | Pentium-4 Willimate Vcore should be auto detected |
|  | Default Northwood (1.5V) | CMOS | Pass | Pentium-4 Northwood Vcore should be auto detected |
|  | User define | CMOS | Pass | Pentium-4 Vcore should be set by user |

| #### Standard CMOS Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  | Test Item | O.S. Environment | Result | Features Tested |
| System Date |  | Date | CMOS | Pass | System data should be within the margin of Spec |
| System Time |  | Time | CMOS | Pass | System time should be within the margin of Spec |
| #### Standard CMOS |  |  |  |  |  |
| Hard Disks | Auto |  | Windows 2K/XP | Pass | IDE HDD auto detection and HDD size >140G |
|  | Manual |  | Windows 2K/XP | Pass | IDE HDD with user-define setting |
|  | None |  | Windows 2K/XP | Pass | With none selected, HDD should not be detected |
| Drive A | 1.44MB |  | Windows 2K/XP | Pass | 1.44Mb Floppy drive |
|  | None |  | Windows 2K/XP | Pass | Disable Floppy drive |
| Drive B | 1.44MB |  | Windows 2K/XP | Pass | 1.44Mb Floppy drive |
|  | None |  | Windows 2K/XP | Pass | Disable Floppy drive |
| Halt On | All Errors |  | CMOS | Pass | Error check must be confirmed since error occurred |
| Install Memory | Base Memory |  | CMOS | Pass | Base memory should match to real memory size |
|  | Extended Memory |  | CMOS | Pass | Extended memory should match to real memory size |
|  | Total Memory |  | CMOS | Pass | Total Memory should match to real memory size |
| #### Advanced BIOS Features |  |  |  |  |  |
| Type |  | Test Item | O.S. Environment | Result | Features Tested |
| Virus Warning |  | Enable | Windows 2K/XP | Pass | BIOS should warn when virus attempt to access the boot sector or partition table |
|  |  | Disable | Windows 2K/XP | Pass | BIOS should not warn when virus attempt to access the boot sector or partition table |
|  |  | Disable | Windows 2K/XP | Pass | Disable L2 Catch |
| Quick POST |  | Enable | CMOS | Pass | BIOS should run completely test procedures |
|  |  | Disable | CMOS | Pass | BIOS should simplify the test procedures |
| Boot Device |  | First | Windows 2K/XP | Pass | O.S. should be boot from first device |
|  |  | Second | Windows 2K/XP | Pass | O.S. should be boot from second device when first device not found |
|  |  | Third | Windows 2K/XP | Pass | O.S. should be boot from third device when first and second device not found |
| Boot Other Device |  | Enable | Windows 2K/XP | Pass | O.S. should be boot from other device |
|  |  | Disable | Windows 2K/XP | Pass | O.S. should not be boot from other device |
| Swap Floppy Drive |  | Enable | CMOS | Pass | CMOS should swap floppy drive a or b |
|  |  | Disable | CMOS | Pass | CMOS should not swap floppy drive a or b |
| Boot up Floppy Seek |  | Enable | CMOS | Pass | CMOS should seek the Floppy Device |
|  |  | Disable | CMOS | Pass | CMOS should not seek the Floppy Device |

| #### Advanced BIOS Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  | Test Item | O.S. Environment | Result | Features Tested |
| Boot up NumLock Status |  | Enable | Windows 2K/XP | Pass | Numlock LED should work while On |
|  |  | Disable | Windows 2K/XP | Pass | Numlock LED should work while On |
| Security Option |  | Setup | CMOS | Pass | Password needs when enter setup menu |
|  |  | System | CMOS | Pass | Password needs when enter setup menu & OS |
| OS select for DRAM |  | Non-OS2 | Windows 2k/XP | Pass | DRAM capacities >64MB should be correct when using non-OS2 operation system |
|  |  | OS2 | OS2 | Pass | DRAM capacities >64MB should be correct when using OS2 operation system |
| Console Redirection |  | Enable | CMOS | Pass | Console redirection function should be enable and work correctly |
|  |  | Disable | CMOS | Pass | Console redirection function should be disable |
| Baud Rate |  | Enable | CMOS | Pass | Baud rate should be set correctly |
| Agent wait time |  | Minutes | CMOS | Pass | Agent wait time should work correctly when enable |
| Agent after boot |  | Enable | CMOS | Pass | Agent should be enable when booting on |
|  |  | Disable | CMOS | Pass | Agent shouldn’t be enable when booting on |
| Video BIOS shadow |  | Enable | CMOS | Pass | Video card BIOS should be shadow to DRAM |
|  |  | Disable | CMOS | Pass | Video card BIOS should not be shadow to DRAM |
| Delay IDE Initial |  | Seconds | CMOS | Pass | BIOS should delay seconds to detect HDD or CD-ROM, DVD-ROM |
| #### Advanced Chipset Features |  |  |  |  |  |
| Type | Test Item |  | O.S. Environment | Result | Features Tested |
| Advance DRAM Control | System Performance |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | DRAM CAS# Latency |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | DDR RAS Precharge Time |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | DDR RAS to CAS Delay |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | DDR Write Recovery time |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | Timing Constraint Control |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | CW Write Mask AGP Request |  | CMOS | Pass | Parameters should set by user according DRAM |
|  | DRAM Driving Strengh |  | CMOS | Pass | Parameters should set by user according DRAM |
| Memory Hole at 15M-16M | Enable |  | Windows 2K/XP | Pass | System should reserve memory hole 15-16M |
|  | Disable |  | Windows 2K/XP | Pass | System should not reserve memory hole 15-16M |
| #### Integrated Peripherals |  |  |  |  |  |
| Type | Test Item |  | O.S. Environment | Result | Features Tested |
| SIS OnChip IDE Device | Internal PCI/IDE |  | Windows 2K/XP | Pass | IDE1 IDE2 should be used by setting |
|  | IDE Burst Mode |  | Windows 2K/XP | Pass | System should use burst mode while enable |
|  | IDE PIO Mode |  | Windows 2K/XP | Pass | System should auto detect IDE device PIO mode |
|  | IDE Ultra DMA |  | Windows 2K/XP | Pass | System should auto detect IDE device ultra DMA mode |
| Onboard Super IO Device | FDD Controller |  | Windows 2K/XP | Pass | Floppy device should be found while enable |
|  | Serial Port 1 |  | Windows 2K/XP | Pass | Serial port should work normally while enable |
|  | Serial Port 2 |  | Windows 2K/XP | Pass | Serial port should work normally while enable |
|  | Parallel Port |  | Windows 2K/XP | Pass | Parallel port should work normally |
|  | Parallel Port Mode |  | Windows 2K/XP | Pass | Different setting of parallel port should work normally |
| USB Controller | Enable |  | Windows 2K/XP | Pass | USB should work normally |
|  | Disable |  | Windows 2K/XP | Pass | USB should not be found by O.S. |
| USB Keyboard Support | OS |  | Windows 2K/XP | Pass | O.S. should use USB keyboard |
|  | BIOS |  | CMOS/DOS | Pass | USB keyboard should be used while system doesn’t support USB |
| IDE HDD Block Mode | Enable |  | Windows 2K/XP | Pass | HDD should use multi-sector transfer |
|  | Disable |  | Windows 2K/XP | Pass | HDD should not use multi-sector transfer |
| Onboard HPT A Controller | Enable |  | Windows 2K/XP | Pass | HPT A should work normally |
|  | Disable |  | Windows 2K/XP | Pass | HPT A should not be found by O.S. |
| Onboard HPT B C D Controller | Enable |  | Windows 2K/XP | Pass | HPT B C D should work normally |
|  | Disable |  | Windows 2K/XP | Pass | HPT B C D should not be found by O.S. |
| Onboard LAN Controller | Enable |  | Windows 2K/XP | Pass | LAN should work normally |
|  | Disable |  | Windows 2K/XP | Pass | LAN should not be found by O.S. |
| IDE Access Interface | PCI Bus |  | Windows 2K/XP | Pass | Access IDE device from PCI bus |
|  | Embedded Bus |  | Windows 2K/XP | Pass | Access IDE device from embedded bus |
| USB1 Access Interface | PCI Bus |  | Windows 2K/XP | Pass | Access IDE device from PCI bus |
|  | Embedded Bus |  | Windows 2K/XP | Pass | Access IDE device from embedded bus |
| USB0 Access Interface | PCI Bus |  | Windows 2K/XP | Pass | Access IDE device from PCI bus |
|  | Embedded Bus |  | Windows 2K/XP | Pass | Access IDE device from embedded bus |

| #### Power management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| ACPI Suspend Type | S1 | Windows 2K/XP | Pass | System should be power on Suspend |
|  | S3 | Windows 2K/XP | Pass | System should be Suspend to RAM |
| Modem Use IRQ | Auto | CMOS | Pass | BIOS should auto apply IRQ for MODEM |
|  | Manual | CMOS | Pass | User should manual apply IRQ for MODEM |
| HDD off After | Time | Windows 2K/XP | Fail | HDD should shut down after setting time |
| Power Button Override | Instant | Windows 2K/XP | Pass | System should be shut down while power button on |
|  | Delay 4 sec | Windows 2K/XP | Fail | System should be shut down while power button on >4 Sec’s |
| Power State Resume Control | Always Off | Windows 2K/XP | Pass | Power state should be off while resume power |
|  | Always On | Windows 2K/XP | Pass | Power state should be on while resume power |
|  | Keep Pre-State | Windows 2K/XP | Pass | Power state should be the same as pre-state while resume power |
| PM Wake Up Events | IRQ [3-7, 9-15] | Windows 2K/XP | Pass | System should be wake up while IRQ [3-7,9-15] be used while enable |
|  | IRQ 8 Break Suspend | Windows 2K/XP | Pass | System should be wake up while IRQ 8 be used while enable |
|  | Wake-Up by PCI Card/ LAN | Windows 2K/XP | Pass | Wake up signal from LAN should awake system while enable |
|  | USB Port wake up control | Windows 2K/XP | Fail | System should be wake up while USB device be used while enable |
|  | KB Power On Password | Windows 2K/XP | Fail | System should be wake up while typing correct password while enable |
|  | Resume by Alarm | CMOS | Fail | System should be wake up while enable |

| #### PnP/PCI Configurations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| Force Update ESCD | Enable | CMOS | Fail | BIOS should update ESCD while boot on every time |
|  | Disable | CMOS | Pass | BIOS should not update while boot on every time |
| Resources Controlled by | Auto (ESCD) | CMOS | Pass | BIOS should auto allocate IRQ to device |
|  | Manual | CMOS | Pass | BIOS should allocate IRQ to device manually |
| PCI/VGA Palette Snoop | Enable | CMOS | Pass | Allow BIOS to preview VGA status while boot on |
|  | Disable | CMOS | Pass | Deny BIOS to preview VGA status while boot on |
| Allocate IRQ to VGA | Enable | CMOS | Pass | BIOS should allocate IRQ to VGA device |
|  | Disable | CMOS | Pass | BIOS should not allocate IRQ to VGA device |
| Allocate TRQ to USB | Enable | CMOS | Pass | BIOS should allocate IRQ to USB device |
|  | Disable | CMOS | Pass | BIOS should not allocate IRQ to USB device |
| PIRQ_0/1 Use IRQ No. | Auto | CMOS | Pass | Allow system to allocate IRQ to PCI slot devices |
|  | Manual | CMOS | Pass | Manual allocate IRQ to PCI slot devices |

| #### PC Health |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| Thermal Monitor | CPU Thermal | CMOS | Pass | Thermal temp of CPU should be reasonable |
|  | System Thermal | CMOS | Pass | Thermal temp of system should be reasonable |
| Fan Monitor | CPU Fan | CMOS | Pass | Fan speed of CPU should be reasonable |
|  | Chassis Fan | CMOS | Pass | Fan speed of chassis should be reasonable |
|  | Power Fan | CMOS | Pass | Fan speed of power should be reasonable |
| Voltage Monitor | VCORE Voltage | CMOS | Pass | VCORE detection should be corrected |
|  | +3.3V Voltage | CMOS | Pass | +3.3V detection should be corrected |
|  | +5V Voltage | CMOS | Pass | +5V detection should be corrected |
|  | +12V Voltage | CMOS | Pass | +12V detection should be corrected |
| #### Exit |  |  |  |  |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| Load Fail-Safe Default | Yes | CMOS | Pass | Load the BIOS values for the most stable, minimal-performance system operations |
|  | No | CMOS | Pass | Don’t load the BIOS values for the most stable, minimal-performance system operations |
| Load Optimized Default | Yes | CMOS | Pass | Load the default values that are factory settings for optimal performance system operations |
|  | No | CMOS | Pass | Don’t load the default values that are factory settings for optimal performance system operations |
| Set Password | Yes | CMOS | Pass | Set password for entering BIOS setup |
|  | No | CMOS | Pass | Don’t set password for entering BIOS setup |
| Save & Exit Setup | Yes | CMOS | Pass | Save BIOS settings & exit |
|  | No | CMOS | Pass | Don’t save BIOS setting & don’t exit |
| Exit without Saving | Yes | CMOS | Pass | Exit without saving |
|  | No | CMOS | Pass | Don’t exit without saving |

| #### Post Screen |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| Screen String | Spelling | CMOS | Pass | Check any unsuitable string during POST |
| DEL Key | Entry BIOS | CMOS | Pass | Del key to entry BIOS should all works any post stage |
| Information List | List of system information | CMOS | Pass | All information in information list should correct |
| IRQ routing table | List of IRQ give | CMOS | Pass | IRQ routing table should be reasonable respect |
| Boot failure | Entry BIOS | CMOS | Pass | System should entry BIOS since last boot failure |

| #### Broadcom PXE BIOS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | Result | Features Tested |
| Boot Protocol | PXE | CMOS | Pass | The system should boot from LAN |
| Boot Strap Type | Auto Detect | CMOS | Pass | The system should auto detect the boot strap type |
|  | BBS | CMOS | Pass | The system should boot from PnP/BBS type |
|  | Int 18h | CMOS | Pass | The system should boot from Int 18h |
|  | Int 19h | CMOS | Pass | The system should boot from Int 19h |
| Hide Setup Prompt | Disable (default) | CMOS | Pass | CMOS should hide the PXE prompt strings |
|  | Enable | CMOS | Pass | CMOS should show the PXE prompt strings |
| Setup Key Stroke | Ctrl-S | CMOS | Pass | The PXE BIOS menu should be entered by the key stroke |
|  | Ctrl-B | CMOS | Pass | The PXE BIOS menu should be entered by the key stroke |
| Banner Message Timeout | Seconds | CMOS | Pass | The banner messages should stay for seconds |
| Link Speed | AutoNeg | CMOS | Pass | Check if auto detect the link speed |
|  | 10Mbps Full/Half | CMOS | Pass | Check if the link speed could be used |
|  | 100Mbps Full/Half | CMOS | Pass | Check if the link speed could be used |
| Preboot Wake On LAN | Enable (default) | CMOS | Pass | PXE BIOS should enable the wake on lan function |
|  | Disable | CMOS | Pass | PXE BIOS should disable the wake on lan function |

  - System Utility Test

| #### System Utility |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Remark |
| Award Flash | V8.01 | Award | DOS | Pass | Flash a new BIOS |
| #### System Diagnostics |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Remark |
| MEMTEST86 | V1.04 | Alexander | N/A | Pass | Execute memory test item |
| Hot CPU Pro | V3.3 |  | W2K | Pass | Execute CPU, chipset, memory test |
| #### System Certification |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Remark |
| Windows 2000 Advance Server HCT | V9.502 | Microsoft | Windows 2000/SP2 | Pass | Execute Motherboard test |
| Windows XP Pro HCT | V10.0 | Microsoft | Windows XP Professional | Pass | Execute Motherboard test |
| Windows 2003 Server Enterprise version | V11.0 | Microsoft | Windows 2003 Server Enterprise | Pass | Execute Motherboard test |
| Linux RedHat | V9.0 | Linux | RedHat | Pass | Installation, Base System, Display, Serial Ports, Parallel Ports, Mouse, Keyboard tests |

  - HPT372 RAID Controller BIOS Function Test

| Item | Function | Hot Spare | Duplicate | Rebuild | Remarks |
| --- | --- | --- | --- | --- | --- |
| Controller 1 (IDE3 IDE4) |  |  |  |  |  |
| RAID 0 | P | N/A | N/A | N/A |  |
| RAID 1 | P | P | P | P |  |
| RAID 0+1 | P | N/A | P | P |  |
| JBOD | P | N/A | N/A | N/A |  |
| Controller 2 (IDE5 IDE6) |  |  |  |  |  |
| RAID 0 | P | N/A | N/A | N/A |  |
| RAID 1 | P | P | P | P |  |
| RAID 0+1 | P | N/A | P | P |  |
| JBOD | P | N/A | N/A | N/A |  |
| Controller 3 (IDE7 IDE8) |  |  |  |  |  |
| RAID 0 | P | N/A | N/A | N/A |  |
| RAID 1 | P | P | P | P |  |
| RAID 0+1 | P | N/A | P | P |  |
| JBOD | P | N/A | N/A | N/A |  |
| Controller 4 (IDE9 IDE10) |  |  |  |  |  |
| RAID 0 | P | N/A | N/A | N/A |  |
| RAID 1 | P | P | P | P |  |
| RAID 0+1 | P | N/A | P | P |  |
| JBOD | P | N/A | N/A | N/A |  |

- Broacom BCM5703 GbE Basic Function Test

  - Basic Function Test

| Item | Functions | OS Environment | Result | Tested Features |
| --- | --- | --- | --- | --- |
| 1 | 10/100 Mbps | Windows 2K/XP | Pass | Check the 10/100 Mbps speed work normally |
| 2 | 1000 Mbps | Windows 2K/XP | Pass | Check the 1000 Mbps speed work normally |
| 3 | Web Browsing | Windows 2K/XP | Pass | Check if IE can brows network |
| 4 | Upload | Windows 2K/XP | Pass | Check if BCM5703 can upload files to file server |
| 5 | Download | Windows 2K/XP | Pass | Check if BCM5703 can download files from files server |
| 6 | Diagnostic tool | Windows 2K/XP | Pass | Check if cable & LEDs functions work done |

  - Advanced Server Program Basic Function & Information

| Item | Functions | OS Environment | Result | Tested Features |
| --- | --- | --- | --- | --- |
| 1 | Fail Over | Windows 2K/XP | Pass | Install 82559 NIC to check if the functions work normally |
| 2 | Link Aggregation | Windows 2K/XP | Pass | Install 82559 NIC to check if the functions work normally |
| 3 | VALN | Windows 2K/XP | Pass | Create VLAN to check if the function work normally |
| 4 | Load Balance | Windows 2K/XP | Pass | Install 82559 NIC to check if the functions work normally |
| 5 | Trunking | Windows 2K/XP | Pass | Install 82559 NIC to check if the functions work normally |

Part B. OPERATION SYSTEM ENVIRONMENT TEST

- Microsoft Windows 98 SE Operation System Test

  - Software Compatibility Test

- Operation System Compatibility

| O.S. Name | Version | Publisher | Status | Test Criteria |
| --- | --- | --- | --- | --- |
| Windows 2003 Server | V 11.0 | Microsoft | Pass | Install all versions of windows 2003 server, and ensure each version is setup successfully and necessary driver is installed. Follow by the testing on software and hardware device, all available connector, jumper, socket and slots are checked. This is to make sure the compatibility and functionality of these features is meet to the specific requirement |
| Windows 2003 Server Enterprise (Chinese Traditional) | V 11.0 | Microsoft | Pass |  |
| Windows 2003 Server web edition | V 11.0 | Microsoft | Pass |  |
| Windows 2003 Server Datacenter edition | V 11.0 | Microsoft | Pass |  |

- Application Software Compatibility

| Database Applications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Visual FoxPro | V6.0 | Microsoft | ##### Windows 2003 Server | Pass | Load a program file, Compile, Link, and Run this program |
| Utilities & Tools |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Norton SystemWork 2002 | V4.0 | Symantec | ##### Windows 2003 Server | Pass | Install and execute virus checking |
| WinZip | V8.1 | Nico Mak | ##### Windows 2003 Server | Pass | Install, compress and run un-compress files |

| Integrated Applications / Project Management |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Office XP | Professional | Microsoft | ##### Windows 2003 Server | Pass | Install from CD-ROM and execute PowerPoint, Excel, Schedule Plus, Word, Access application sample files open and save function |
| Office 2000 | Professional | Microsoft | ##### Windows 2003 Server | Pass | Install from CD-ROM and execute PowerPoint, Excel, Schedule Plus, Word, Access application sample files open and save function |
| Programming Language / Compiler |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Visual Basic | V6.0 | Microsoft | ##### Windows 2003 Server | Pass | Install from CD-ROM, Load Sample execute it |
| Visual C++ | V6.0 | Microsoft | ##### Windows 2003 Server | Pass | Install from CD-ROM, Load Sample execute it |
| Visual InterDev | V6.0 | Microsoft | ##### Windows 2003 Server | Pass | Install from CD-ROM, Load Sample execute it |
| Visual Studio 2003 Enterprise Architecture | V7.0 | Microsoft | ##### Windows 2003 Server | Pass | Install From DVD-ROM, Load Sample, execute it |
| Communication / Remote Control / E-mail |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Microsoft Internet Explorer | V6.0 | Microsoft | ##### Windows 2003 Server | Pass | Down/Up load files via a FTP web server |
| Microsoft Outlook Express | V6.0 | Microsoft | ##### Windows 2003 Server | Pass | Receive/Send E-mail |
| Media Player |  |  |  |  |  |
| Power DVD XP | V4.0 | CyberLink | ##### Windows 2003 Server | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |

Supporting Software Compatibility

| Support CD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Version | Publisher | ##### Status | Features Tested |
| Autorun Test | N/A | Akom | ##### Pass | CD inserts or directly run Autorun program |
| Award Flash Test | V8.01 | Award | ##### Pass | Run flash.exe to update BIOS |
| SIS IDE Utility |  | SIS | ##### Pass | Install utility and check all available functions |
| Hardware Monitor |  | Winbond | ##### Pass | Install and check all available functions |
| Broadcom BACS |  | Broadcom | ##### Pass | Install utility |
| HPT Management |  | HighPoint | ##### Pass | Install utility and check all available functions |
| Brower CD | N/A | Akom | ##### Pass | Brower directory, check any unsuitable content |
| Manual | N/A | Akom | ##### Pass | Open manual. Check its content |
| Exit | N/A | Akom | ##### Pass | Exit Autorun Menu |

System Special Function Compatibility

| Special Functions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Direct Cable Connection | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Direct cable data transferring test via existing COM port, Parallel port |
| Windows 2003 System Suspend Test | 2003 | Akom | ##### Windows 2003 Server | Pass | With existing Win2000, add auto suspend to startup field, aging overnight test |
| Windows 2003 System Rebooting | 2003 | Akom | ##### Windows 2003 Server | Pass | With existing , add auto reboot to setup field, aging overnight test |
| Windows 2003 System Shut down | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Shut down O.S. and see its work normally |
| Windows 2003 System Information Checking | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Check all items working pass under system, information listing |
| Multiple Language Support | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Implement multiple language installation on pure English edition |
| Unattended Installation | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Implement unattended installation |
| Full loading DRAM | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Build up and aging system with full DRAM installed |
| Full loading IDE | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Build up and aging system with full PCI devices installed |
| Full loading USB | 2003 | Microsoft | ##### Windows 2003 Server | Pass | Build up and aging system with full USB devices installed |

| System Tools and Strategy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Function | O.S. Environment | ##### Status | Features Tested |
| Hard Disk | File system | Windows 2003 Server | ##### Pass | Install Windows 2000 in any available file system |
|  | Upgrade to Dynamic | Windows 2003 Server | ##### Pass | Upgrade basic to dynamic HDD |
|  | Disk Clear up | Windows 2003 Server | ##### Pass | Clear unused system files |
|  | Scandisk | Windows 2003 Server | ##### Pass | Scan and fix HDD error |
|  | Defragment | Windows 2003 Server | ##### Pass | HDD defragment |
|  | Quota set | Windows 2003 Server | ##### Pass | Set HDD using limit |
|  | Compress drive | Windows 2003 Server | ##### Pass | Compress files |
|  | EFS | Windows 2003 Server | ##### Pass | Encrypted files |
| Shut Down Control | Log off | Windows 2003 Server | ##### Pass | Logging off current user |
|  | Shut down | Windows 2003 Server | ##### Pass | Shut down system |
|  | Suspend | Windows 2003 Server | ##### Pass | Suspending system |
|  | Hibernate | Windows 2003 Server | ##### Pass | Hibernating system |
|  | Power key to shut down | Windows 2003 Server | ##### Pass | Direct power key to shut down system |
|  | Power key to suspend | Windows 2003 Server | ##### Pass | Direct power key to suspending system |
|  | K/B power key | Windows 2003 Server | ##### Pass | Keyboard power key to shut down system |
|  | K/B sleep key | Windows 2003 Server | ##### Pass | Keyboard sleep key to suspend system |
| Timer Automatic work | Timer monitor off | Windows 2003 Server | ##### Pass | Set timer to monitor off |
|  | Timer HDD off | Windows 2003 Server | ##### Pass | Set timer to HDD off |
|  | Timer suspend system | Windows 2003 Server | ##### Pass | Set timer to suspending system |
|  | Timer hibernate system | Windows 2003 Server | ##### Pass | Set timer to hibernating system |
|  | Timer screen saver | Windows 2003 Server | ##### Pass | Set timer to run screen saver |
|  | Scheduled task | Windows 2003 Server | ##### Pass | Schedule timer to run task |

| System Tools and Strategy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Function |  | O.S. Environment | ##### Status | Features Tested |
| System Upgrading | DC Promotion |  | Windows 2003 Server | ##### Pass | Promote serer to domain controller |
| Network services and Application | DNS |  | Windows 2003 Server | ##### Pass | Install and run DNS |
|  | Terminal service |  | Windows 2003 Server | ##### Pass | Implement terminal service |
|  | DHCP |  | Windows 2003 Server | ##### Pass | Install and run DHCP |
|  | WINS |  | Windows 2003 Server | ##### Pass | Install and run WINS |
|  | NAT |  | Windows 2003 Server | ##### Pass | Implement NAT service |
|  | DFS |  | Windows 2003 Server | ##### Pass | Implement and run DFS |
| RAID Function | RAID-0 |  | Windows 2003 Server | ##### Pass | Implement RAID-0 function to combine IDE3 to IDE10 HDDs |
|  | RAID-1 |  | Windows 2003 Server | ##### Pass | Implement RAID-1 to duplicate HDD |
|  | RAID-5 |  | Windows 2003 Server | ##### Pass | Implement RAID-5 function to combine IDE3 to IDE10 HDDs |
| HPT372 Hot Swap | Hardware RAID | No RAID | Windows 2003 Server | Pass | Online Hot Swap HPT372 HDD |
|  |  | RAID-1 | Windows 2003 Server | Pass | Online Hot Swap HPT372 HDD and restore RAID-1 |
|  |  | RAID-0+1 | Windows 2003 Server | Pass | O-line Hot Swap HPT372 HDD and restore RAID-0+1 |
|  | Software RAID | RAID-1 | Windows 2003 Server | Pass | Online Hot Swap HPT372 HDD and restore RAID-1 |
|  |  | RAID-5 | Windows 2003 Server | Pass | Online Hot Swap HPT372 HDD and restore RAID-5 |
|  | Hardware RAID + Software RAID | RAID-10 | Windows 2003 Server | Pass | Online Hot Swap HPT372 and restore RAID-10 |

| ACPI BIOS Test |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  | Test Item |  |  | O.S. Environment |  |  | ##### Result |  |  | Features Tested |
| Timer Power Saving |  | Monitor off |  |  | Windows 2003 Server |  |  | Pass |  |  | Set timer for monitor off power saving |
|  |  | HDD off |  |  | Windows 2003 Server |  |  | Pass |  |  | Set timer for HDD off power saving |
|  |  | Standby |  |  | Windows 2003 Server |  |  | Pass |  |  | Set time for Standby power saving |
| Power Button |  | Power off in ACPI mode |  |  | Windows 2003 Server |  |  | Pass |  |  | The system soft off |
|  |  | Stand by in ACPI mode |  |  | Windows 2003 Server |  |  | Pass |  |  | The system goes standby mode |
| Power work |  | Power supply fan |  |  | Windows 2003 Server |  |  | ##### Pass |  |  | Check all available fan status while system entry S3 |
| FAN work |  | All on board fan |  |  | Windows 2003 Server |  |  | ##### Pass |  |  | Check all available fan status while system entry S3 |
| External Signal to Wake up (S1) |  | ACPI wake on keyboard |  |  | Windows 2003 Server |  |  | Pass |  |  | Given a signal via Keyboard to wake up a standby system |
|  |  | ACPI wake up on PS/2 Mouse |  |  | Windows 2003 Server |  |  | Pass |  |  | Given a signal via PS/2 Mouse to wake up a standby system |
| External Signal to Wake up (S3) |  | ACPI wake up on keyboard |  |  | Windows 2003 Server |  |  | Pass |  |  | Press power button to wake up a standby system |
|  |  | ACPI wake up on PS/2 Mouse |  |  | Windows 2003 Server |  |  | Pass |  |  | Given a signal via Keyboard to wake up a standby system |
|  |  | ACPI wake up on Power Button |  |  | Windows 2003 Server |  |  | Pass |  |  | Press power button to wake up a standby system |
| External Signal to Power up (S5) |  | ACPI power on keyboard |  |  | Windows 2003 Server |  |  | Pass |  |  | Given a signal via Keyboard to power up a standby system |
|  |  | ACPI power on PS/2 Mouse |  |  | Windows 2003 Server |  |  | Pass |  |  | Given a signal via PS/2 Mouse to power up a standby system |
|  |  | ACPI power on Power Button |  |  | Windows 2003 Server |  |  | Pass |  |  | Press power button to power up a standby system |
| Broadcom BACS Sever Function Test |  |  |  |  |  |  |  |  |  |  |  |
| Type | Test Item |  |  | O.S. Environment |  |  | Result |  |  | Features Tested |  |
| Server Functions |  |  | Fail Over |  |  | Windows 2003 Server | Pass |  | Install 82559NIC to check if the functions work normally |  |  |
|  |  |  | Link Aggregation |  |  | Windows 2003 Server | Pass |  | Install 82559NIC to check if the functions work normally |  |  |
|  |  |  | VALN |  |  | Windows 2003 Server | Pass |  | Create VLAN to check if the function work normally |  |  |
|  |  |  | Load Balance |  |  | Windows 2003 Server | Pass |  | Install 82559NIC to check if the functions work normally |  |  |
|  |  |  | Trunking |  |  | Windows 2003 Server | Pass |  | Install 82559NIC to check if the functions work normally |  |  |

  - Hardware Compatibility Test

- System Device Functionality Test

| IDE controller Mode detection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Item | Version | Publisher | ##### OS Environment | Status | Features Tested |
| IDE1 | System | SIS | ##### Windows 2003 Server | Pass | Check IDE1 mode detection |
| IDE2 | System | SIS | ##### Windows 2003 Server | Pass | Check IDE2 mode detection |
| IDE3 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE3 mode detection |
| IDE4 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE4 mode detection |
| IDE5 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE5 mode detection |
| IDE6 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE6 mode detection |
| IDE7 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE7 mode detection |
| IDE8 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE8 mode detection |
| IDE9 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE9 mode detection |
| IDE10 | V2.32 | HighPoint | ##### Windows 2003 Server | Pass | Check IDE10 mode detection |

| IDE controller |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item | HDD/CD-ROM | ##### M/S | OS Environment | Status |
| IDE1 | Maxtor D740X-6L (ATA 133) | ##### M | Windows 2003 Server | Pass |
| IDE2 | Lite-On LTN-163 (DVD 16X) | ##### M | Windows 2003 Server | Pass |
| IDE3 | Seagate 340016A (ATA 100) | ##### M | Windows 2003 Server | Pass |
| IDE4 | Maxtor D740X-6L (ATA 133) | ##### M | Windows 2003 Server | Pass |
| IDE5 | IBM IC35L160AVVA070 (160G) | ##### M | Windows 2003 Server | Pass |
| IDE6 | Seagate 320011A | ##### M | Windows 2003 Server | Pass |
| IDE7 | IBM DTLA-307045 | ##### M | Windows 2003 Server | Pass |
| IDE8 | IBM IC35L120AVVA070 (120G) | ##### M | Windows 2003 Server | Pass |
| IDE9 | WD 7200RPM 120G | ##### M | Windows 2003 Server | Pass |
| IDE10 | WD 7200RPM 160G | ##### M | Windows 2003 Server | Pass |

- Addon device functionality and Capatility

| Graphic Adapters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Lemel 6326 | ##### OS PnP | Windows 2003 Server | Pass | SIS6326 |
|  | Lemel 305 | ##### V3.0 | Windows 2003 Server | Pass | SIS305 |
|  | Trident Blad3D | ##### OS PnP | Windows 2003 Server | Pass | Blade 3D |
| 100 Base Tx Ethernet Adapters |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Intel Pro 100 | ##### OS PnP | Windows 2003 Server | Pass | I82559 |
|  | Intel Pro100S | ##### V6.1.3.0 | Windows 2003 Server | Pass | I82550 |
|  | 3C980C-TXM | ##### V3.48.0.0 | Windows 2003 Server | Pass | Broadcom |
|  | Lemel RTL8139 | ##### V4.0.1.2 | Windows 2003 Server | Pass | RTL8139C |
| SCSI Adapter |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Adaptec 2100S | ##### V3.5.1 | Windows 2003 Server | Pass | Adaptec |
|  | Adaptec 3960D | ##### V3.6.2.1 | Windows 2003 Server | Pass | Adpatec |
| Floppy Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| Normal | NEC FD 123 H | ##### FDD | Windows 2003 Server | Pass |  |
|  | Mitumi D353M3D | ##### FDD | Windows 2003 Server | Pass |  |
|  | Panasonic JU-257A606P | ##### FDD | Windows 2003 Server | Pass |  |
| USB 1.1 | Acer YD-8U10-01 | ##### USB1 2 | Windows 2003 Server | Pass |  |
| ISDN Modem Device |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Remark |
| Serial | ZyXEL ISDN Terminal Adapter Omni TA 128 | ##### System | Windows 2003 Server | Pass |  |
| Dial-Up Modem Device |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Remark |
| Serial | Mitac E210 | ##### V1.01 | Windows 2003 Server | Pass |  |
|  | GVC FM-56JT | ##### V1.06b | Windows 2003 Server | Pass |  |
| PCI | PCTel HSP56 Micro Modem | ##### V7.65.0003.0 | Windows 2003 Server | Pass |  |
| USB 1.1 | Mitac E500 | ##### V2.84.00 | Windows 2003 Server | Pass |  |

| Mouse Device |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| Serial | Logitec M-MM43 | ##### COM1, COM2 | Windows 2003 Server | Pass |  |
|  | Microsoft Basic Mouse PS/2 | ##### COM1 | Windows 2003 Server | Pass |  |
| USB 1.1 | Logitec M-BD53 | ##### USB1 2 | Windows 2003 Server | Pass |  |
|  | Logitec M-BD58 | ##### USB1 2 | Windows 2003 Server | Pass |  |
|  | Genius NetScroll+ NMU+P | ##### USB1 2 | Windows 2003 Server | Pass |  |
|  | Microsoft IntelliMouse | ##### USB1 2 | Windows 2003 Server | Pass |  |
|  | Microsoft IntelliMouse Explorer | ##### USB1 2 | Windows 2003 Server | Pass |  |
|  | Microsoft IntelliMouse Optical | ##### USB1 2 | Windows 2003 Server | Pass |  |
| PS/2 | Logitec M-CAB48A | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Logitec M-S34 | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Logitec M-S43 | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Genius PS/2 Mouse | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Microsoft Basic Mouse PS/2 | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | A4Tech WinEasy WWW-5 | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Microsoft IntelliMouse | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Microsoft Wheel Mouse Optical | ##### PS/2 | Windows 2003 Server | Pass |  |

| Keyboard Devices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| PS/2 | Microsoft Internet Keyboard | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Microsoft Natural Keyboard Pro | ##### PS/2 | Windows 2003 Server | Pass |  |
|  | Logitec Internet Keyboard | ##### PS/2 | Windows 2003 Server | Pass |  |
| CD-R/RW Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | YAMAHA CRW8424E | ##### IDE1 IDE2 | Windows 2003 Server | Pass |  |
|  | ASUS CRW-1210R | ##### IDE1 IDE2 | Windows 2003 Server | Pass |  |
| USB1.1 | Iomega ZIPCD1024EXT | ##### USB1 2 | Windows 2003 Server | Pass |  |
| DVD-R.DVD-RW Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | Ricoh MP5120A | ##### IDE1 2 | Windows 2003 Server | Pass |  |
| ZIP Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | Iomega Z100 ATAP13 | ##### IDE1 IDE2 | Windows 2003 Server | Pass |  |
|  | Iomega Z250 ATAPI250 | ##### IDE1 IDE2 | Windows 2003 Server | Pass |  |
| USB 1.1 | Iomega Z250USBPCMBP | ##### USB1 2 | Windows 2003 Server | Pass |  |

  - Field Stress Test

| - Test Type | - Windows 2003 Server Multitasking with network access test |  |  |
| --- | --- | --- | --- |
| ##### Configuration | ##### Intel Pentium 4-2000 ##### 768MB (DDR266 SDRAM 256MB*3) IDE3 toIDE10 Seagate ST340016A IDE1 IBM IC35L120AVVA070 IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Multitasking on local PC and access remote workstation resource |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |
| - Test Type | - Windows 2003 Server Hot CPU Pro 3.3 |  |  |
| ##### Configuration | Intel Pentium 4-2400 768MB (DDR333 SDRAM 256MB*3) IDE3 to IDE10 Seagate ST340016A IDE1 Maxtor D740X-6L IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Hot CPU Pro 3.3 burn-in test |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |
| - Test Type | - Windows 2003 Server 3D Mark2002 Demo Mode |  |  |
| Configuration | Intel Pentium 4-2200 768MB (DDR333 SDRAM 256MB*3) IDE3 to IDE10 Seagate ST340016A IDE1 Seagate ST320011A IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run 3D Mark2002 Demo Mode |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |

- Microsoft Windows 2000 Operation System Test

  - Software Compatibility Test

- Operation System Compatibility

| O.S. Name | Version | Publisher | Status | Test Criteria |
| --- | --- | --- | --- | --- |
| Windows 2000 Professional | V2195 | Microsoft | Pass | Install all versions of windows NT, and ensure each version is setup successfully and necessary driver is installed. Follow by the testing on software and hardware device, all available connector, jumper, socket and slots are checked. This is to make sure the compatibility and functionality of these features is meet to the specific requirement |
| Windows 2000 Server | V2195 | Microsoft | Pass |  |
| Windows 2000 Advance Server | V2195 | Microsoft | Pass |  |
| Windows 2000 Professional (Chinese Traditional) | V2195 | Microsoft | Pass |  |
| Windows 2000 Server (Chinese Traditional) | V2195 | Microsoft | Pass |  |
| Windows 2000 Advance Server (Chinese Traditional) | V2195 | Microsoft | Pass |  |

- Application Software Compatibility

| Database Applications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Borland dBase7 | V7.0 | Borland | ##### Windows2000 AD Server | Pass | Load from CD-ROM, and using capture and image edition function |
| SQL Server | V7.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Build up SQL server, create and access data |
| Visual FoxPro | V6.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Load a program file, Compile, Link, and Run this program |
| Utilities & Tools |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| PC Anywhere | V10.0 | Symantec | ##### Windows 2000 Professional | Pass | Build up anywhere utility, remote access and control through network |
| Norton SystemWork 2002 | 2002 | Symantec | ##### Windows 2000 Professional | Pass | Install and execute virus checking |
| WinZip | V8.1 | Nico Mak | ##### Windows 2000 AD Server | Pass | Install, compress and run un-compress files |

| Integrated Applications / Project Management |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Project 2000 | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Load a sample file and make organize project |
| Office XP | 2001 | Microsoft | ##### Windows 2000 AD Server | Pass | Install from CD-ROM and execute PowerPoint, Excel, Schedule Plus, Word, Access application sample files open and save function |
| Office 2000 | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Install from CD-ROM and execute PowerPoint, Excel, Schedule Plus, Word, Access application sample files open and save function |
| Programming Language / Compiler |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Visual Studio .Net | V7.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Install from CD-ROM, Load Sample executive |
| Visual Basic | V6.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Install from CD-ROM, Load Sample execute it |
| Visual C++ | V6.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Install from CD-ROM, Load Sample execute it |
| Visual InterDev | V6.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Install from CD-ROM, Load Sample execute it |
| Communication / Remote Control / E-mail |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Microsoft Internet Explorer | V6.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Down/Up load files via a FTP web server |
| Microsoft Outlook Express | V6.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Receive/Send E-mail |
| Media Player |  |  |  |  |  |
| Power DVD XP | V4.0 | CyberLink | ##### Windows 2000 AD Server | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |
| Microsoft Media Player | V7.0 | Microsoft | ##### Windows 2000 AD Server | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |
| ASUS DVD 2000 Player | V3.0 | ASUS | ##### Windows 2000 AD Server | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |

Supporting Software Compatibility

| Support CD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Version | Publisher | ##### Status | Features Tested |
| Autorun Test | None | Akom | ##### Pass | CD inserts or directly run Autorun program |
| Award Flash Test | V8.01 | Award | ##### Pass | Run flash.exe to update BIOS |
| SIS IDE Utility |  | SIS | ##### Pass | Install utility and check all available functions |
| Hardware Monitor |  | Winbond | ##### Pass | Install and check all available functions |
| Broadcom BACS |  | Broadcom | ##### Pass | Install utility |
| HPT Management |  | HighPoint | ##### Pass | Install utility and check all available functions |
| Brower CD | N/A | Akom | ##### Pass | Brower directory, check any unsuitable content |
| Manual | N/A | Akom | ##### Pass | Open manual. Check its content |
| Exit | N/A | Akom | ##### Pass | Exit Autorun Menu |

System Special Function Compatibility

| Special Functions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Direct Cable Connection | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Direct cable data transferring test via existing COM port, Parallel port |
| Windows 2000 System Suspend Test | 2000 | Akom | ##### Windows 2000 AD Server | Pass | With existing Win2000, add auto suspend to startup field, aging overnight test |
| Windows 2000 System Rebooting | 2000 | Akom | ##### Windows 2000 AD Server | Pass | With existing , add auto reboot to setup field, aging overnight test |
| Windows 2000 System Shut down | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Shut down O.S. and see its work normally |
| Windows 2000System Information Checking | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Check all items working pass under system, information listing |
| Multiple Language Support | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Implement multiple language installation on pure English edition |
| Service Pack | 2 | Microsoft | ##### Windows 2000 AD Server | Pass | Implement service pack installation |
| Unattended Installation | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Implement unattended installation |
| Full loading DRAM | 2000 | Microsoft | ##### Windows 2000 AD Server | Pass | Build up and aging system with full DRAM installed |

| Special Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package Name |  | Version | Publisher |  | ##### OS Environment |  | Status |  | Features Tested |
| Full loading IDE |  | 2000 | Microsoft |  | ##### Windows 2000 AD Server |  | Pass |  | Build up and aging system with full PCI devices installed |
| Full loading USB |  | 2000 | Microsoft |  | ##### Windows 2000 AD Server |  | Pass |  | Build up and aging system with full USB devices installed |
| System Tools and Strategy |  |  |  |  |  |  |  |  |  |
| Type | Function |  |  | O.S. Environment |  | ##### Status |  | Features Tested |  |
| Hard Disk | File system |  |  | Windows 2000 AD Server |  | ##### Pass |  | Install Windows 2000 in any available file system |  |
|  | Upgrade to Dynamic |  |  | Windows 2000 AD Server |  | ##### Pass |  | Upgrade basic to dynamic HDD |  |
|  | Disk Clear up |  |  | Windows 2000 AD Server |  | ##### Pass |  | Clear unused system files |  |
|  | Scandisk |  |  | Windows 2000 AD Server |  | ##### Pass |  | Scan and fix HDD error |  |
|  | Defragment |  |  | Windows 2000 AD Server |  | ##### Pass |  | HDD defragment |  |
|  | Quota set |  |  | Windows 2000 AD Server |  | ##### Pass |  | Set HDD using limit |  |
|  | Compress drive |  |  | Windows 2000 AD Server |  | ##### Pass |  | Compress files |  |
|  | EFS |  |  | Windows 2000 AD Server |  | ##### Pass |  | Encrypted files |  |

| System Tools and Strategy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Function | O.S. Environment | ##### Status | Features Tested |
| Shut Down Control | Log off | Windows 2000 AD Server | ##### Pass | Logging off current user |
|  | Shut down | Windows 2000 AD Server | ##### Pass | Shut down system |
|  | Suspend | Windows 2000 AD Server | ##### Pass | Suspending system |
|  | Hibernate | Windows 2000 AD Server | ##### Pass | Hibernating system |
|  | Power key to shut down | Windows 2000 AD Server | ##### Pass | Direct power key to shut down system |
|  | Power key to suspend | Windows 2000 AD Server | ##### Pass | Direct power key to suspending system |
|  | K/B power key | Windows 2000 AD Server | ##### Pass | Keyboard power key to shut down system |
|  | K/B sleep key | Windows 2000 AD Server | ##### Pass | Keyboard sleep key to suspend system |
| Timer Automatic work | Timer monitor off | Windows 2000 AD Server | ##### Pass | Set timer to monitor off |
|  | Timer HDD off | Windows 2000 AD Server | ##### Pass | Set timer to HDD off |
|  | Timer suspend system | Windows 2000 AD Server | ##### Pass | Set timer to suspending system |
|  | Timer hibernate system | Windows 2000 AD Server | ##### Pass | Set timer to hibernating system |
|  | Timer screen saver | Windows 2000 AD Server | ##### Pass | Set timer to run screen saver |
|  | Scheduled task | Windows 2000 AD Server | ##### Pass | Schedule timer to run task |
| System Upgrading | DC Promotion | Windows 2000 AD Server | ##### Pass | Promote serer to domain controller |

| System Tools and Strategy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Function |  | O.S. Environment | ##### Status | Features Tested |
| Network services and Application | DNS |  | Windows 2000 AD Server | ##### Pass | Install and run DNS |
|  | Terminal service |  | Windows 2000 AD Server | ##### Pass | Implement terminal service |
|  | DHCP |  | Windows 2000 AD Server | ##### Pass | Install and run DHCP |
|  | WINS |  | Windows 2000 AD Server | ##### Pass | Install and run WINS |
|  | NAT |  | Windows 2000 AD Server | ##### Pass | Implement NAT service |
|  | DFS |  | Windows 2000 AD Server | ##### Pass | Implement and run DFS |
| RAID Function | RAID-0 |  | Windows 2000 AD Server | ##### Pass | Implement RAID-0 function to combine IDE3 to IDE10 HDDs |
|  | RAID-1 |  | Windows 2000 AD Server | ##### Pass | Implement RAID-1 to duplicate HDD |
|  | RAID-5 |  | Windows 2000 AD Server | ##### Pass | Implement RAID-5 function to combine IDE3 to IDE10 HDDs |
| HPT372 Hot Swap | Hardware RAID | No RAID | Windows 2000 AD Server | Pass | Online Hot Swap HPT372 HDD |
|  |  | RAID-1 | Windows 2000 AD Server | Pass | Online Hot Swap HPT372 HDD and restore RAID-1 |
|  |  | RAID-0+1 | Windows 2000 AD Server | Pass | O-line Hot Swap HPT372 HDD and restore RAID-0+1 |
|  | Software RAID | RAID-1 | Windows 2000 AD Server | Pass | Online Hot Swap HPT372 HDD and restore RAID-1 |
|  |  | RAID-5 | Windows 2000 AD Server | Pass | Online Hot Swap HPT372 HDD and restore RAID-5 |
|  | Hardware RAID + Software RAID | RAID-10 | Windows 2000 AD Server | Pass | On=line Hot Swap HPT372 and restore RAID-10 |

| ACPI BIOS Test |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Test Item | O.S. Environment | ##### Result | Features Tested |
| Timer Power Saving | Monitor off | Windows 2000 AD Server | Pass | Set timer for monitor off power saving |
|  | HDD off | Windows 2000 AD Server | Pass | Set timer for HDD off power saving |
|  | Standby | Windows 2000 AD Server | Pass | Set time for Standby power saving |
| Power Button | Power off in ACPI mode | Windows 2000 AD Server | Pass | The system soft off |
|  | Stand by in ACPI mode | Windows 2000 AD Server | Pass | The system goes standby mode |
| Power work | Power supply fan | Windows 2000 AD Server | ##### Pass | Check all available fan status while system entry S3 |
| FAN work | All on board fan | Windows 2000 AD Server | ##### Pass | Check all available fan status while system entry S3 |
| External Signal to Wake up (S1) | ACPI wake on keyboard | Windows 2000 AD Server | Pass | Given a signal via Keyboard to wake up a standby system |
|  | ACPI wake up on PS/2 Mouse | Windows 2000 AD Server | Pass | Given a signal via PS/2 Mouse to wake up a standby system |
| External Signal to Wake up (S3) | ACPI wake up on keyboard | Windows 2000 AD Server | Pass | Press power button to wake up a standby system |
|  | ACPI wake up on PS/2 Mouse | Windows 2000 AD Server | Pass | Given a signal via Keyboard to wake up a standby system |
|  | ACPI wake up on Power Button | Windows 2000 AD Server | Pass | Press power button to wake up a standby system |
| External Signal to Power up (S5) | ACPI power on keyboard | Windows 2000 AD Server | Pass | Given a signal via Keyboard to power up a standby system |
|  | ACPI power on PS/2 Mouse | Windows 2000 AD Server | Pass | Given a signal via PS/2 Mouse to power up a standby system |
|  | ACPI power on Power Button | Windows 2000 AD Server | Pass | Press power button to power up a standby system |

| Broadcom BACS Sever Function Test |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Test Item |  | O.S. Environment |  | Result |  | Features Tested |
| Server Functions |  | Fail Over |  | Windows 2000 AD Server | Pass | Install 82559NIC to check if the functions work normally |  |
|  |  | Link Aggregation |  | Windows 2000 AD Server | Pass | Install 82559NIC to check if the functions work normally |  |
|  |  | VALN |  | Windows 2000 AD Server | Pass | Create VLAN to check if the function work normally |  |
|  |  | Load Balance |  | Windows 2000 AD Server | Pass | Install 82559NIC to check if the functions work normally |  |
|  |  | Trunking |  | Windows 2000 AD Server | Pass | Install 82559NIC to check if the functions work normally |  |

  - Hardware Compatibility Test

- System Device Functionality Test

| IDE controller Mode detection |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Item | Version | Publisher | ##### OS Environment | Status | Features Tested |  |
| IDE1 | System | SIS | ##### Windows 2000 AD Server | Pass | Check IDE1 mode detection |  |
| IDE2 | System | SIS | ##### Windows 2000 AD Server | Pass | Check IDE2 mode detection |  |
| IDE3 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE3 mode detection |  |
| IDE4 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE4 mode detection |  |
| IDE5 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE5 mode detection |  |
| IDE6 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE6 mode detection |  |
| IDE7 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE7 mode detection |  |
| IDE8 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE8 mode detection |  |
| IDE9 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE9 mode detection |  |
| IDE10 | V2.32 | HighPoint | ##### Windows 2000 AD Server | Pass | Check IDE10 mode detection |  |
| IDE controller |  |  |  |  |  |  |
| Test Item | HDD/CD-ROM |  | ##### M/S | OS Environment |  | Status |
| IDE1 | Maxtor D740X-6L (ATA 133) |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE2 | Lite-On LTN-163 (DVD 16X) |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE3 | Seagate 340016A (ATA 100) |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE4 | Maxtor D740X-6L (ATA 133) |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE5 | IBM IC35L160AVVA070 (160G) |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE6 | Seagate 320011A |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE7 | IBM DTLA-307045 |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE8 | IBM IC35L120AVVA070 (120G) |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE9 | WD 7200RPM 120G |  | ##### M | Windows 2000 AD Server |  | Pass |
| IDE10 | WD 7200RPM 160G |  | ##### M | Windows 2000 AD Server |  | Pass |

- Addon device functionality and Ca[atility

| Graphic Adapters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Lemel 6326 | ##### OS PnP | Windows 2000 AD Server | Pass | SIS6326 |
|  | Lemel 305 | ##### V3.0 | Windows 2000 AD Server | Pass | SIS305 |
|  | Trident Blad3D | ##### OS PnP | Windows 2000 AD Server | Pass | Blade 3D |
| 100 Base Tx Ethernet Adapters |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Intel Pro 100 | ##### OS PnP | Windows 2000 AD Server | Pass | I82559 |
|  | Intel Pro100S | ##### V6.1.3.0 | Windows 2000 AD Server | Pass | I82550 |
|  | 3C980C-TXM | ##### V3.48.0.0 | Windows 2000 AD Server | Pass | Broadcom |
|  | Lemel RTL8139 | ##### V4.0.1.2 | Windows 2000 AD Server | Pass | RTL8139C |
|  | D-Link DFE-530TX | ##### V2.66.0.290 | Windows 2000 AD Server | Pass | Davicom DM9101F |
|  | Accton EN1207F-TX | ##### V1.12.10.16 | Windows 2000 AD Server | Pass | Accton EN5251BE |
|  | Buffalo LGY-PCI-TXL | ##### V2.11.2000.5 | Windows 2000 AD Server | Pass | MX MX98715AEC-E |
| SCSI Adapter |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Adaptec 2100S | ##### V3.5.1 | Windows 2000 AD Server | Pass | Adaptec |
|  | Adaptec 3960D | ##### V3.6.2.1 | Windows 2000 AD Server | Pass | Adpatec |
| Floppy Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| Normal | NEC FD 123 H | ##### FDD | Windows 2000 AD Server | Pass |  |
|  | Mitumi D353M3D | ##### FDD | Windows 2000 AD Server | Pass |  |
|  | Panasonic JU-257A606P | ##### FDD | Windows 2000 AD Server | Pass |  |
| USB 1.1 | Acer YD-8U10-01 | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
| ISDN Modem Device |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Remark |
| Serial | ZyXEL ISDN Terminal Adapter Omni TA 128 | ##### System | Windows 2000 AD Server | Pass |  |

| Dial-Up Modem Device |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Driver Version | OS Environment | Status | Remark |
| Serial | Mitac E210 | ##### V1.01 | Windows 2000 AD Server | Pass |  |
|  | GVC FM-56JT | ##### V1.06b | Windows 2000 AD Server | Pass |  |
| PCI | PCTel HSP56 Micro Modem | ##### V7.65.0003.0 | Windows 2000 AD Server | Pass |  |
| USB 1.1 | Mitac E500 | ##### V2.84.00 | Windows 2000 AD Server | Pass |  |
| Mouse Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| Serial | Logitec M-MM43 | ##### COM1, COM2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft Basic Mouse PS/2 | ##### COM1 | Windows 2000 AD Server | Pass |  |
| USB 1.1 | Logitec M-BD53 | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
|  | Logitec M-BD58 | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
|  | Genius NetScroll+ NMU+P | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft IntelliMouse | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft IntelliMouse Explorer | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft IntelliMouse Optical | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
| PS/2 | Logitec M-CAB48A | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Logitec M-S34 | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Logitec M-S43 | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Genius PS/2 Mouse | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft Basic Mouse PS/2 | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | A4Tech WinEasy WWW-5 | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft IntelliMouse | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft Wheel Mouse Optical | ##### PS/2 | Windows 2000 AD Server | Pass |  |
| Keyboard Devices |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| PS/2 | Microsoft Internet Keyboard | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Microsoft Natural Keyboard Pro | ##### PS/2 | Windows 2000 AD Server | Pass |  |
|  | Logitec Internet Keyboard | ##### PS/2 | Windows 2000 AD Server | Pass |  |
| CD-R/RW Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | YAMAHA CRW8424E | ##### IDE1 IDE2 | Windows 2000 AD Server | Pass |  |
|  | ASUS CRW-1210R | ##### IDE1 IDE2 | Windows 2000 AD Server | Pass |  |
| USB1.1 | Iomega ZIPCD1024EXT | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
| DVD-R.DVD-RW Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | Ricoh MP5120A | ##### IDE1 2 | Windows 2000 AD Server | Pass |  |
| ZIP Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | Iomega Z100 ATAP13 | ##### IDE1 IDE2 | Windows 2000 AD Server | Pass |  |
|  | Iomega Z250 ATAPI250 | ##### IDE1 IDE2 | Windows 2000 AD Server | Pass |  |
| USB 1.1 | Iomega Z250USBPCMBP | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
| Printer Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| USB 1.1 | Epson Perfection 1640SU Photo | ##### USB1 2 | Windows 2000 AD Server | Pass |  |
| Parallel | HP Deskjet 990cxi | ##### LPT | Windows 2000 AD Server | Pass |  |

  - Field (Stress) Test

| - Test Type | - Windows 2000 Advance Server Iometer Disk Stress Test |  |  |
| --- | --- | --- | --- |
| ##### Configuration | ##### Intel Pentium 4-2000 ##### 768MB (DDR266 SDRAM 256MB*3) IDE3 toIDE10 Seagate ST340016A IDE1 IBM IC35L120AVVA070 IDE2 Teac CD-540E Software RAID-5 Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Iometer 1999.10.20 to stress RAID disk partitions |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |
| - Test Type | - Windows 2000 Advance Server Iometer Network Stress Test |  |  |
| ##### Configuration | ##### Intel Pentium 4-2000 ##### 768MB (DDR266 SDRAM 256MB*3) IDE3 toIDE10 Seagate ST340016A IDE1 IBM IC35L120AVVA070 IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Iometer 1999.10.20 to connect 5 clients for stressing BCM5703 gigabit NIC |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |

| - Test Type | - Windows 2000 Server C.C. Winstone Test |  |  |
| --- | --- | --- | --- |
| ##### Configuration | Intel Pentium 4-2400 768MB (DDR333 SDRAM 256MB*3) IDE3 to IDE10 Seagate ST340016A IDE1 Maxtor D740X-6L IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run C.C. Winstone 20002 CCWS test suite |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 50 times |
| - Test Type | - Windows 2000 Server System Winmark 2002 Test |  |  |
| ##### Configuration | Intel Pentium 4-2400 768MB (DDR333 SDRAM 256MB*3) IDE3 to IDE10 Seagate ST340016A IDE1 Maxtor D740X-6L IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run System Winmark 20002 all test suite |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 50 times |

- Microsoft Windows XP Operation System Test

  - Software Compatibility Test

- Operation System Compatibility

| O.S. Name | Version | Publisher | Status | Test Criteria |
| --- | --- | --- | --- | --- |
| Windows XP Home | 2002 | Microsoft | Pass | Install all versions of windows NT, and ensure each version is setup successfully and necessary driver is installed. Follow by the testing on software and hardware device, all available connector, jumper, socket and slots are checked. This is to make sure the compatibility and functionality of these features is meet to the specific requirement |
| Windows 2000 XP Professional | 2002 | Microsoft | Pass |  |
| Windows XP Home (Chinese Traditional) | 2002 | Microsoft | Pass |  |
| Windows XP Professional (Chinese Traditional) | 2002 | Microsoft | Pass |  |

- Application Software Compatibility

| Image Processing |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher |  | ##### OS Environment |  | Status |  | Features Tested |
| ACD See | V4.0 | ACD system |  | ##### WinXP Pro. |  | Pass |  | Install from CD-ROM, and Opening picture |
| Visio 2002 | 2002 | Microsoft |  | ##### WinXP Pro |  | Pass |  | Install, open sample file and edit it |
| Dreammeaver | V4.0 | Macromedia |  | ##### WinXP Home |  | Pass |  | Install, open sample file and edit it |
| Database Applications |  |  |  |  |  |  |  |  |
| Package Name | Version | Publisher |  | ##### OS Environment | Status |  |  | Features Tested |
| Borland dBase7 | V7.0 | Borland |  | ##### WinXP Pro | Pass |  |  | Load from CD-ROM, and using capture and image edition function |
| SQL Server | V7.0 | Microsoft |  | ##### WinXP .Net Server | Pass |  |  | Build up SQL server, create and access data |
| Visual FoxPro | V6.0 | Microsoft |  | ##### WinXP Pro | Pass |  |  | Load a program file, Compile, Link, and Run this program |
| Utilities & Tools |  |  |  |  |  |  |  |  |
| Package Name | Version |  | Publisher | ##### OS Environment | Status |  | Features Tested |  |
| PC Anywhere | V11.0 |  | Symantec | ##### WinXP Pro | Pass |  | Build up anywhere utility, remote access and control through network |  |
| Norton Antivirus | 2003 |  | Symantec | ##### WinXP Pro | Pass |  | Executive virus checking |  |
| WinZip | V8.1 |  | Nico Mak | ##### WinXP Pro | Pass |  | Install, compress and run un-compress files |  |

| Integrated Applications / Project Management |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Project 2000 | 2000 | Microsoft | ##### WinXP Pro | Pass | Load a sample file and make organize project |
| Office XP | XP | Microsoft | ##### WinXP Pro (Chinese Traditional) | Pass | Install from CD-ROM and execute PowerPoint, Excel, Schedule Plus, Word, Access application sample files open and save function |
| Office 2000 | 2000 | Microsoft | ##### WinXP Pro (Chinese Traditional) | Pass | Install from CD-ROM and execute PowerPoint, Excel, Schedule Plus, Word, Access application sample files open and save function |
| Programming Language / Compiler |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Visual Studio .Net | V7.0 | Microsoft | ##### WinXP Pro | Pass | Install from CD-ROM, Load Sample executive |
| Visual Basic | V6.0 | Microsoft | ##### WinXP Pro | Pass | Install from CD-ROM, Load Sample execute it |
| Visual C++ | V6.0 | Microsoft | ##### WinXP Pro | Pass | Install from CD-ROM, Load Sample execute it |
| Visual InterDev | V6.0 | Microsoft | ##### WinXP Pro | Pass | Install from CD-ROM, Load Sample execute it |
| Communication / Remote Control / E-mail |  |  |  |  |  |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Microsoft Internet Explorer | V6.0 | Microsoft | ##### WinXP Pro | Pass | Down/Up load files via a FTP web server |
| Microsoft Outlook Express | V6.0 | Microsoft | ##### WinXP Pro | Pass | Receive/Send E-mail |
| Media Player |  |  |  |  |  |
| Power DVD XP | V4.0 | CyberLink | ##### WinXP Pro | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |
| Microsoft Media Player | V7.0 | Microsoft | ##### WinXP Pro | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |
| WinDVD XP | V4.0 | CyberLink | ##### WinXP Pro | Pass | Open an existing DVD format CD from DVD-ROM, execute player with different size of windows open |

Supporting Software Compatibility

| Support CD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Version | Publisher | ##### Status | Features Tested |
| Autorun Test | None | Akom | ##### Pass | CD inserts or directly run Autorun program |
| Award Flash Test | V8.01 | Award | ##### Pass | Run flash.exe to update BIOS |
| SIS IDE Utility |  | SIS | ##### Pass | Install utility and check all available functions |
| Hardware Monitor |  | Winbond | ##### Pass | Install and check all available functions |
| Broadcom BACS |  | Broadcom | ##### Pass | Install utility |
| HPT Management |  | HighPoint | ##### Pass | Install utility and check all available functions |
| Brower CD | N/A | Akom | ##### Pass | Brower directory, check any unsuitable content |
| Manual | N/A | Akom | ##### Pass | Open manual. Check its content |
| Exit | N/A | Akom | ##### Pass | Exit Autorun Menu |

System Special Function Compatibility

| Special Functions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | ##### OS Environment | Status | Features Tested |
| Direct Cable Connection | 2000 | Microsoft | ##### Win XP Pro | Pass | Direct cable data transferring test via existing COM port, Parallel port |
| Windows 2000 System Suspend Test | 2000 | Akom | ##### Win XP Pro | Pass | With existing Win2000, add auto suspend to startup field, aging overnight test |
| Windows 2000 System Rebooting | 2000 | Akom | ##### Win XP Pro | Pass | With existing , add auto reboot to setup field, aging overnight test |
| Windows 2000 System Shut down | 2000 | Microsoft | ##### Win XP Pro | Pass | Shut down O.S. and see its work normally |
| Windows 2000System Information Checking | 2000 | Microsoft | ##### Win XP Pro | Pass | Check all items working pass under system, information listing |
| Multiple Language Support | 2000 | Microsoft | ##### Win XP Pro | Pass | Implement multiple language installation on pure English edition |
| Service Pack | 1 | Microsoft | ##### Win XP Pro | Pass | Implement service pack installation |
| Unattended Installation | 2000 | Microsoft | ##### Win XP Pro | Pass | Implement unattended installation |
| Full loading DRAM | 2000 | Microsoft | ##### Win XP Pro | Pass | Build up and aging system with full DRAM installed |
| Full loading IDE | 2000 | Microsoft | ##### Win XP Pro | Pass | Build up and aging system with full PCI devices installed |
| Full loading USB | 2000 | Microsoft | ##### Win XP Pro | Pass | Build up and aging system with full USB devices installed |

| System Tools and Strategy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Function | O.S. Environment | ##### Status | Features Tested |
| Hard Disk | File system | ##### Win XP Pro | ##### Pass | Install Windows 2000 in any available file system |
|  | Upgrade to Dynamic | ##### Win XP Pro | ##### Pass | Upgrade basic to dynamic HDD |
|  | Disk Clear up | ##### Win XP Pro | ##### Pass | Clear unused system files |
|  | Scandisk | ##### Win XP Home | ##### Pass | Scan and fix HDD error |
|  | Defragment | ##### Win XP Pro | ##### Pass | HDD defragment |
|  | Quota set | ##### Win XP Pro | ##### Pass | Set HDD using limit |
|  | Compress drive | ##### Win XP Pro | ##### Pass | Compress files |
|  | EFS | ##### Win XP Pro | ##### Pass | Encrypted files |
| Shut Down Control | Log off | ##### Win XP Pro | ##### Pass | Logging off current user |
|  | Shut down | ##### Win XP Pro | ##### Pass | Shut down system |
|  | Suspend | ##### Win XP Pro | ##### Pass | Suspending system |
|  | Hibernate | ##### Win XP Pro | ##### Pass | Hibernating system |
|  | Power key to shut down | ##### Win XP Pro | ##### Pass | Direct power key to shut down system |
|  | Power key to suspend | ##### Win XP Pro | ##### Pass | Direct power key to suspending system |
|  | K/B power key | ##### Win XP Pro | ##### Pass | Keyboard power key to shut down system |
|  | K/B sleep key | ##### Win XP Pro | ##### Pass | Keyboard sleep key to suspend system |
| Timer Automatic work | Timer monitor off | ##### Win XP Pro | ##### Pass | Set timer to monitor off |
|  | Timer HDD off | ##### Win XP Pro | ##### Pass | Set timer to HDD off |
|  | Timer suspend system | ##### Win XP Pro | ##### Pass | Set timer to suspending system |
|  | Timer hibernate system | ##### Win XP Pro | ##### Pass | Set timer to hibernating system |
|  | Timer screen saver | ##### Win XP Pro | ##### Pass | Set timer to run screen saver |
|  | Scheduled task | ##### Win XP Home | ##### Pass | Schedule timer to run task |
| System Upgrading | DC Promotion | ##### Win XP Pro | ##### Pass | Promote serer to domain controller |
| Network services and Application | DNS | ##### Win XP Pro | Pass | Install and run DNS |
|  | Terminal service | ##### Win XP Pro | Pass | Implement terminal service |
|  | DHCP | ##### Win XP Pro | Pass | Install and run DHCP |
|  | WINS | ##### Win XP Pro | Pass | Install and run WINS |
|  | NAT | ##### Win XP Pro | Pass | Implement NAT service |
|  | DFS | ##### Win XP Pro | Pass | Implement and run DFS |

| System Tools and Strategy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Function |  |  |  |  | O.S. Environment |  |  |  | ##### Status |  |  | Features Tested |  |  |  |
| RAID Function | RAID-0 |  |  |  |  | ##### Win XP Pro |  |  |  | Pass |  |  | Implement RAID-0 function to combine IDE3 to IDE10 HDDs |  |  |  |
|  | RAID-1 |  |  |  |  | ##### Win XP Pro |  |  |  | Pass |  |  | Implement RAID-1 to duplicate HDD |  |  |  |
|  | RAID-5 |  |  |  |  | ##### Win XP Pro |  |  |  | Pass |  |  | Implement RAID-5 function to combine IDE3 to IDE10 HDDs |  |  |  |
| HPT372 Hot Swap | Hardware RAID |  |  |  | No RAID | ##### Win XP Pro |  |  |  | Pass |  |  | Online Hot Swap HPT372 HDD |  |  |  |
|  |  |  |  |  | RAID-1 | ##### Win XP Pro |  |  |  | Pass |  |  | Online Hot Swap HPT372 HDD and restore RAID-1 |  |  |  |
|  |  |  |  |  | RAID-0+1 | ##### Win XP Pro |  |  |  | Pass |  |  | O-line Hot Swap HPT372 HDD and restore RAID-0+1 |  |  |  |
|  | Software RAID |  |  |  | RAID-1 | ##### Win XP Pro |  |  |  | Pass |  |  | Online Hot Swap HPT372 HDD and restore RAID-1 |  |  |  |
|  |  |  |  |  | RAID-5 | Win XP Pro |  |  |  | Pass |  |  | Online Hot Swap HPT372 HDD and restore RAID-5 |  |  |  |
|  | Hardware RAID + Software RAID |  |  |  | RAID-10 | Win XP Pro |  |  |  | Pass |  |  | On=line Hot Swap HPT372 and restore RAID-10 |  |  |  |
| ACPI BIOS Test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  | Test Item |  |  |  |  | O.S. Environment |  |  |  | ##### Result |  |  |  | Features Tested |
| Timer Power Saving |  |  | Monitor off |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Set timer for monitor off power saving |
|  |  |  | HDD off |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Set timer for HDD off power saving |
|  |  |  | Standby |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Set time for Standby power saving |
| Power Button |  |  | Power off in ACPI mode |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | The system soft off |
|  |  |  | Stand by in ACPI mode |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | The system goes standby mode |
| Power work |  |  | Power supply fan |  |  |  |  | Win XP Pro |  |  |  | ##### Pass |  |  |  | Check all available fan status while system entry S3 |
| FAN work |  |  | All on board fan |  |  |  |  | Win XP Pro |  |  |  | ##### Pass |  |  |  | Check all available fan status while system entry S3 |
| External Signal to Wake up (S1) |  |  | ACPI wake on keyboard |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Given a signal via Keyboard to wake up a standby system |
|  |  |  | ACPI wake up on PS/2 Mouse |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Given a signal via PS/2 Mouse to wake up a standby system |
| External Signal to Wake up (S3) |  |  | ACPI wake up on keyboard |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Press power button to wake up a standby system |
|  |  |  | ACPI wake up on PS/2 Mouse |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Given a signal via Keyboard to wake up a standby system |
|  |  |  | ACPI wake up on Power Button |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Press power button to wake up a standby system |
| External Signal to Power up (S5) |  |  | ACPI power on keyboard |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Given a signal via Keyboard to power up a standby system |
|  |  |  | ACPI power on PS/2 Mouse |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Given a signal via PS/2 Mouse to power up a standby system |
|  |  |  | ACPI power on Power Button |  |  |  |  | Win XP Pro |  |  |  | Pass |  |  |  | Press power button to power up a standby system |
| Broadcom BACS Sever Function Test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  | Test Item |  |  |  |  | O.S. Environment |  |  |  | Result |  |  |  | Features Tested |  |
| Server Functions |  |  |  | Fail Over |  |  |  |  | Win XP Pro |  | Pass |  |  | Install 82559NIC to check if the functions work normally |  |  |
|  |  |  |  | Link Aggregation |  |  |  |  | Win XP Pro |  | Pass |  |  | Install 82559NIC to check if the functions work normally |  |  |
|  |  |  |  | VALN |  |  |  |  | Win XP Pro |  | Pass |  |  | Create VLAN to check if the function work normally |  |  |
|  |  |  |  | Load Balance |  |  |  |  | Win XP Pro |  | Pass |  |  | Install 82559NIC to check if the functions work normally |  |  |
|  |  |  |  | Trunking |  |  |  |  | Win XP Pro |  | Pass |  |  | Install 82559NIC to check if the functions work normally |  |  |

  - Hardware Compatibility Test

- System Device Functionality Test

| IDE controller Mode detection |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Item | Version | Publisher | ##### OS Environment | Status | Features Tested |  |
| IDE1 | System | SIS | ##### Win XP Pro | Pass | Check IDE1 mode detection |  |
| IDE2 | System | SIS | ##### Win XP Pro | Pass | Check IDE2 mode detection |  |
| IDE3 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE3 mode detection |  |
| IDE4 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE4 mode detection |  |
| IDE5 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE5 mode detection |  |
| IDE6 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE6 mode detection |  |
| IDE7 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE7 mode detection |  |
| IDE8 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE8 mode detection |  |
| IDE9 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE9 mode detection |  |
| IDE10 | V2.32 | HighPoint | ##### Win XP Pro | Pass | Check IDE10 mode detection |  |
| IDE controller |  |  |  |  |  |  |
| Test Item | HDD/CD-ROM |  | ##### M/S | OS Environment |  | Status |
| IDE1 | Maxtor D740X-6L (ATA 133) |  | ##### M | Win XP Pro |  | Pass |
| IDE2 | Lite-On LTN-163 (DVD 16X) |  | ##### M | Win XP Pro |  | Pass |
| IDE3 | Seagate 340016A (ATA 100) |  | ##### M | Win XP Pro |  | Pass |
| IDE4 | Maxtor D740X-6L (ATA 133) |  | ##### M | Win XP Pro |  | Pass |
| IDE5 | IBM IC35L160AVVA070 (160G) |  | ##### M | Win XP Pro |  | Pass |
| IDE6 | Seagate 320011A |  | ##### M | Win XP Pro |  | Pass |
| IDE7 | IBM DTLA-307045 |  | ##### M | Win XP Pro |  | Pass |
| IDE8 | IBM IC35L120AVVA070 (120G) |  | ##### M | Win XP Pro |  | Pass |
| IDE9 | WD 7200RPM 120G |  | ##### M | Win XP Pro |  | Pass |
| IDE10 | WD 7200RPM 160G |  | ##### M | Win XP Pro |  | Pass |

- Addon device functionality and Ca[atility

| Graphic Adapters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Lemel 6326 | ##### OS PnP | Win XP Pro | Pass | SIS6326 |
|  | Lemel 305 | ##### V3.0 | Win XP Pro | Pass | SIS305 |
|  | Trident Blad3D | ##### OS PnP | Win XP Pro | Pass | Blade 3D |
| 100 Base Tx Ethernet Adapters |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Intel Pro 100 | ##### OS PnP | Win XP Pro | Pass | I82559 |
|  | Intel Pro100S | ##### V6.1.3.0 | Win XP Pro | Pass | I82550 |
|  | 3C980C-TXM | ##### V3.48.0.0 | Win XP Pro | Pass | Broadcom |
|  | Lemel RTL8139 | ##### V4.0.1.2 | Win XP Pro | Pass | RTL8139C |
|  | D-Link DFE-530TX | ##### V2.66.0.290 | Win XP Pro | Pass | Davicom DM9101F |
|  | Accton EN1207F-TX | ##### V1.12.10.16 | Win XP Pro | Pass | Accton EN5251BE |
|  | Buffalo LGY-PCI-TXL | ##### V2.11.2000.5 | Win XP Pro | Pass | MX MX98715AEC-E |
| SCSI Adapter |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Chipset |
| PCI | Adaptec 2100S | ##### V3.5.1 | Win XP Pro | Pass | Adaptec |
|  | Adaptec 3960D | ##### V3.6.2.1 | Win XP Pro | Pass | Adaptec |
| Floppy Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| Normal | NEC FD 123 H | ##### FDD | Win XP Pro | Pass |  |
|  | Mitumi D353M3D | ##### FDD | Win XP Pro | Pass |  |
|  | Panasonic JU-257A606P | ##### FDD | Win XP Pro | Pass |  |
| USB 1.1 | Acer YD-8U10-01 | ##### USB1 2 | Win XP Pro | Pass |  |
| ISDN Modem Device |  |  |  |  |  |
| Type | Model | ##### Driver Version | OS Environment | Status | Remark |
| Serial | ZyXEL ISDN Terminal Adapter Omni TA 128 | ##### System | Win XP Pro | Pass |  |

| Dial-Up Modem Device |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Driver Version | OS Environment | Status | Remark |
| Serial | Mitac E210 | ##### V1.01 | Win XP Pro | Pass |  |
|  | GVC FM-56JT | ##### V1.06b | Win XP Pro | Pass |  |
| PCI | PCTel HSP56 Micro Modem | ##### V7.65.0003.0 | Win XP Pro | Pass |  |
| USB 1.1 | Mitac E500 | ##### V2.84.00 | Win XP Pro | Pass |  |
| Mouse Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| Serial | Logitec M-MM43 | ##### COM1, COM2 | Win XP Pro | Pass |  |
|  | Microsoft Basic Mouse PS/2 | ##### COM1 | Win XP Pro | Pass |  |
| USB 1.1 | Logitec M-BD53 | ##### USB1 2 | Win XP Pro | Pass |  |
|  | Logitec M-BD58 | ##### USB1 2 | Win XP Pro | Pass |  |
|  | Genius NetScroll+ NMU+P | ##### USB1 2 | Win XP Home | Pass |  |
|  | Microsoft IntelliMouse | ##### USB1 2 | Win XP Pro | Pass |  |
|  | Microsoft IntelliMouse Explorer | ##### USB1 2 | Win XP Pro | Pass |  |
|  | Microsoft IntelliMouse Optical | ##### USB1 2 | Win XP Pro | Pass |  |
| PS/2 | Logitec M-CAB48A | ##### PS/2 | Win XP Pro | Pass |  |
|  | Logitec M-S34 | ##### PS/2 | Win XP Pro | Pass |  |
|  | Logitec M-S43 | ##### PS/2 | Win XP Pro | Pass |  |
|  | Genius PS/2 Mouse | ##### PS/2 | Win XP Home | Pass |  |
|  | Microsoft Basic Mouse PS/2 | ##### PS/2 | Win XP Pro | Pass |  |
|  | A4Tech WinEasy WWW-5 | ##### PS/2 | Win XP Pro | Pass |  |
|  | Microsoft IntelliMouse | ##### PS/2 | Win XP Pro | Pass |  |
|  | Microsoft Wheel Mouse Optical | ##### PS/2 | Win XP Pro | Pass |  |

| Keyboard Devices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| PS/2 | Microsoft Internet Keyboard | ##### PS/2 | Win XP Pro | Pass |  |
|  | Microsoft Natural Keyboard Pro | ##### PS/2 | Win XP Pro | Pass |  |
|  | Logitec Internet Keyboard | ##### PS/2 | Win XP Pro | Pass |  |
| CD-R/RW Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | YAMAHA CRW8424E | ##### IDE1 IDE2 | Win XP Pro | Pass |  |
|  | ASUS CRW-1210R | ##### IDE1 IDE2 | Win XP Pro | Pass |  |
| USB1.1 | Iomega ZIPCD1024EXT | ##### USB1 2 | Win XP Pro | Pass |  |
| DVD-R.DVD-RW Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | Ricoh MP5120A | ##### IDE1 2 | Win XP Pro | Pass |  |
| ZIP Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| IDE | Iomega Z100 ATAP13 | ##### IDE1 IDE2 | Win XP Pro | Pass |  |
|  | Iomega Z250 ATAPI250 | ##### IDE1 IDE2 | Win XP Pro | Pass |  |
| USB 1.1 | Iomega Z250USBPCMBP | ##### USB1 2 | Win XP Pro | Pass |  |
| Printer Device |  |  |  |  |  |
| Type | Model | ##### Connect Interface | OS Environment | Status | Remark |
| USB 1.1 | Epson Perfection 1640SU Photo | ##### USB1 2 | Win XP Pro | Pass |  |
| Parallel | HP Deskjet 990cxi | ##### LPT | Win XP Pro | Pass |  |

  - Field (Stress) Test

| - Test Type | - Windows XP .Net Server Iometer Disk Stress Test |  |  |
| --- | --- | --- | --- |
| ##### Configuration | ##### Intel Pentium 4-2000 ##### 768MB (DDR266 SDRAM 256MB*3) IDE3 toIDE10 Seagate ST340016A IDE1 IBM IC35L120AVVA070 IDE2 Teac CD-540E Software RAID-5 Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Iometer 1999.10.20 to stress RAID disk partitions |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |
| - Test Type | - Windows XP .Net Server Iometer Network Stress Test |  |  |
| ##### Configuration | ##### Intel Pentium 4-2000 ##### 768MB (DDR266 SDRAM 256MB*3) IDE3 toIDE10 Seagate ST340016A IDE1 IBM IC35L120AVVA070 IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Iometer 1999.10.20 to connect 5 clients for stressing BCM5703 gigabit NIC |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |

| - Test Type | - Windows XP Pro C.C. Winstone 2002 Test |  |  |
| --- | --- | --- | --- |
| ##### Configuration | Intel Pentium 4-2400 768MB (DDR333 SDRAM 256MB*3) IDE3 to IDE10 Seagate ST340016A IDE1 Maxtor D740X-6L IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run C.C. Winstone 20002 CCWS test suite |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 50 times |
| - Test Type | - Windows XP Pro System Winmark 2002 Test |  |  |
| ##### Configuration | Intel Pentium 4-2400 768MB (DDR333 SDRAM 256MB*3) IDE3 to IDE10 Seagate ST340016A IDE1 Maxtor D740X-6L IDE2 Teac CD-540E Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run System Winmark 20002 all test suite |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 50 times |

- Linux Operation System Test

  - Software Compatibility Test

| O.S. Name | Version | ##### Publisher | Status | Test Criteria |
| --- | --- | --- | --- | --- |
| RedHat Linux | V7.3 | ##### RedHat | Pass | Install different versions of Linux, and ensure each version is set up successfully. Follow by the testing on hardware device and make sure each device is functional and compatible |
| RedHat Linux | V8.0 | ##### RedHat | Pass |  |
| RedHat Linux | V9.0 | ##### RedHat | Pass |  |

  - Hardware Compatibility Test

| IDE controller Mode detection |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Item |  | Version |  | Publisher | ##### OS Environment |  | Status | Features Tested |  |  |  |
| IDE1 |  | System |  | SIS | ##### Redhat V9.0 |  | Pass | Check IDE1 mode detection |  |  |  |
| IDE2 |  | System |  | SIS | ##### Redhat V9.0 |  | Pass | Check IDE2 mode detection |  |  |  |
| IDE3 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE3 mode detection |  |  |  |
| IDE4 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE4 mode detection |  |  |  |
| IDE5 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE5 mode detection |  |  |  |
| IDE6 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE6 mode detection |  |  |  |
| IDE7 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE7 mode detection |  |  |  |
| IDE8 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE8 mode detection |  |  |  |
| IDE9 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE9 mode detection |  |  |  |
| IDE10 |  | V2.32 |  | HighPoint | ##### Redhat V9.0 |  | Pass | Check IDE10 mode detection |  |  |  |
| IDE controller |  |  |  |  |  |  |  |  |  |  |  |
| Test Item |  | HDD/CD-ROM |  |  | ##### M/S |  | OS Environment |  |  |  | Status |
| IDE1 |  | Maxtor D740X-6L (ATA 133) |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE2 |  | Lite-On LTN-163 (DVD 16X) |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE3 |  | Seagate 340016A (ATA 100) |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE4 |  | Maxtor D740X-6L (ATA 133) |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE5 |  | IBM IC35L160AVVA070 (160G) |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE6 |  | Seagate 320011A |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE7 |  | IBM DTLA-307045 |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE8 |  | IBM IC35L120AVVA070 (120G) |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE9 |  | WD 7200RPM 120G |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| IDE10 |  | WD 7200RPM 160G |  |  | ##### M |  | Redhat V9.0 |  |  |  | Pass |
| SCSI Adapter |  |  |  |  |  |  |  |  |  |  |  |
| Type | Model |  | ##### Driver Version |  |  | OS Environment |  |  | Status | Chipset |  |
| PCI | Adaptec 2100S |  | ##### Redhat V9.0 |  |  | Redhat V9.0 |  |  | Pass | Adaptec |  |
|  | Adaptec 3960D |  | ##### Redhat V9.0 |  |  | Redhat V9.0 |  |  | Pass | Adaptec |  |

  - Field (stress) Test for Networking Environment

| - Test Type | - RedHat Linux V7.3 Multitasking with network access test |  |  |
| --- | --- | --- | --- |
| ##### Configuration | ##### Intel Pentium 4-2000 ##### 768MB (DDR266 SDRAM 256MB*3) IDE3 toIDE10 Seagate ST340016A IDE1 IBM IC35L120AVVA070 IDE2 Teac CD-540E Software RAID-5 Logitec Keyboard and Mouse |  |  |
| ##### Test Procedure | ##### Run Multi Tasks Test, Ping, Telnet…….test |  |  |
| ##### Result | ##### Pass | ##### Duration | ##### 72 Hours |

Part C. Performance Measurement

Hardware Environment:

M/B: AK-SR7

PCB Version: V0.21

CPU:  Intel Pentium 4-2000 MHz (100* 20), L2 Catch 512KB

DRAM: 512MB (DDR333 SDRAM) KingMax DDR333 PBGA

Storage controller: On-board IDE controller

RAID controller: On-board HPT372 RAID controller

Storage HDD: IDE Maxtor ATA133 7200RPM D740X-6L

VGA: Lemel SIS6326 8MB SDRAM

Pointer: Logitec PS/2 Optical Mouse

Keyboard: Logitec PS/2 keyboard

Software and BIOS environment:

BIOS Version: SR7_0.9

BIOS Setting: Optimized Defaults

VGA Driver Version: OS PnP

VGA Resolution: 800*600*32bit, 75 Hz refresh rate, small font

O.S.: Windows 2000 Advance Server/SP2

- Disk I/O Performance Test

Iometer 1999.10.20 (disk)

RAID-0

| Access Specification Name |  | Workers | IOps | MBps | Average Response Time |
| --- | --- | --- | --- | --- | --- |
| 64K | Sequential 100% Read | 20 | 1599.08 | 99.94 | 12.50 |
| 512K | Sequential 100% Read | 20 | 215.84 | 107.92 | 92.60 |
| File Server | Iometer內建 | 20 | 586.46 | 6.34 | 34.09 |

RAID-5

| Access Specification Name |  | Workers | IOps | MBps | Average Response Time |
| --- | --- | --- | --- | --- | --- |
| 64K | Sequential 100% Read | 20 | 1648.27 | 103.02 | 12.13 |
| 512K | Sequential 100% Read | 20 | 203.50 | 101.75 | 98.28 |
| File Server | Iometer內建 | 20 | 428.45 | 4.60 | 46.65 |

RAID-10 (Hardware RAID-1 + Software RAID-0)

| Access Specification Name |  | Workers | IOps | MBps | Average Response Time |
| --- | --- | --- | --- | --- | --- |
| 64K | Sequential 100% Read | 20 | 826.21 | 51.64 | 24.20 |
| 512K | Sequential 100% Read | 20 | 196.28 | 98.14 | 101.84 |
| File Server | Iometer內建 | 20 | 580.87 | 6.30 | 34.42 |

ZD Winbench 99 V2.0 (Disk)

Software RAID 0 (8 Disks)

| Disk Transfer Rate |  | High-End Disk Winmark | Business Disk Winmark |
| --- | --- | --- | --- |
| Beginning | End |  |  |
| 128000 | 126000 | 42600 | 13700 |

Software RAID 5 (8 Disks)

| Disk Transfer Rate |  | High-End Disk Winmark | Business Disk Winmark |
| --- | --- | --- | --- |
| Beginning | End |  |  |
| 119000 | 116000 | 18500 | 6400 |

Hardware RAID 1 + Software RAID 0 ( Total 8 Disks)

| Disk Transfer Rate |  | High-End Disk Winmark | Business Disk Winmark |
| --- | --- | --- | --- |
| Beginning | End |  |  |
| 104000 | 102000 | 41900 | 19200 |

B. Network I/O Performance Test

Iometer 1999.10.20 (Network)

|  | IOps | Read IOps | Write IOps | MBps | Read MBps | Write MBps |
| --- | --- | --- | --- | --- | --- | --- |
| 2K-Sends | 46090.40 | 0.00 | 46090.40 | 90.02 | 0.00 | 90.02 |
| 4K-Sends | 26011.06 | 0.00 | 26011.06 | 101.61 | 0.00 | 101.61 |
| 8K-Sends | 13603.01 | 0.00 | 13603.01 | 106.27 | 0.00 | 106.27 |
| 16K-Sends | 6958.03 | 0.00 | 6958.03 | 108.72 | 0.00 | 108.72 |
| 32K-Sends | 3525.05 | 0.00 | 3525.05 | 110.16 | 0.00 | 110.16 |
| 64K-Sends | 1776.63 | 0.00 | 1776.63 | 111.04 | 0.00 | 111.04 |
| 128K-Sends | 895.67 | 0.00 | 895.67 | 111.96 | 0.00 | 111.96 |
| 256K-Sends | 450.86 | 0.00 | 450.86 | 112.72 | 0.00 | 112.72 |
| 2K-Receives | 38095.87 | 38095.87 | 0.00 | 74.41 | 74.41 | 0.00 |
| 4K-Receives | 25521.88 | 25521.88 | 0.00 | 99.69 | 99.69 | 0.00 |
| 8K-Receives | 13558.27 | 13558.27 | 0.00 | 105.92 | 105.92 | 0.00 |
| 16K-Receives | 6899.60 | 6899.60 | 0.00 | 107.81 | 107.81 | 0.00 |
| 32K-Receives | 3606.27 | 3606.27 | 0.00 | 112.70 | 112.70 | 0.00 |
| 64]K-Receives | 1803.30 | 1803.30 | 0.00 | 112.71 | 112.71 | 0.00 |
| 128K-Receives | 902.35 | 902.35 | 0.00 | 112.79 | 112.79 | 0.00 |
| 256K-Receives | 451.81 | 451.81 | 0.00 | 112.95 | 112.95 | 0.00 |
| 2K-Mix | 31589.43 | 15802.01 | 15787.42 | 61.70 | 30.86 | 30.83 |
| 4K-Mix | 22803.40 | 11407.96 | 11395.44 | 89.08 | 44.56 | 44.51 |
| 8K-Mix | 14772.91 | 7379.10 | 7393.82 | 115.41 | 57.65 | 57.76 |
| 16K-Mix | 8531.79 | 4261.46 | 4270.33 | 133.31 | 66.59 | 66.72 |
| 32K-Mix | 4385.77 | 2190.81 | 2194.96 | 137.06 | 68.46 | 68.59 |
| 64K-Mix | 2174.59 | 1091.29 | 1083.31 | 135.91 | 68.21 | 67.71 |
| 128K-Mix | 1081.21 | 542.69 | 538.52 | 135.15 | 67.84 | 67.31 |
| 256K-Mix | 541.54 | 270.15 | 271.39 | 135.38 | 67.54 | 67.85 |

Server Configurations

| CPU | Intel P4-2.0AG |
| --- | --- |
| M/B | AK-SR7 V0.21 |
| RAM | 512MB DDRRAM266 DIMM1 |
| NIC | On board Broadcom BCM5703 A2 |
| VGA | Sis6326 8MB SDRAM PCI Slot1 |
| OS HD | Seagate 20G 7200RPM 2MB Catch |

Client Configurations

| Client 1 |  |
| --- | --- |
| CPU | Intel P4-1.4G |
| M/B | Abit BW7 |
| RAM | 256MB SDRAM PC-133 DIMM1 |
| NIC | 3Com 3C996 GbE |
| OS HD | IBM 45G 7200RPM 2MB Catch |
| Client 2 |  |
| CPU | Intel P4-1.6AG |
| M/B | Abit BL7 |
| RAM | 256MB SDRAM PC-133 DIMM1 |
| NIC | Akom BCM5701 GbE |
| OS HD | IBM 40G 7200 RPM 2MB Catch |
| Client 3 |  |
| CPU | Intel P4-1.4G |
| M/B | Abit TH7 |
| RAM | 256MB RDRAM PC-800 SIMM1 SIMM2 |
| NIC | Intel PRO/1000 T |
| OS HD | Seagate 40G 7200RPM 2MB Catch |
| Client 4 |  |
| CPU | Intel P4-1.4G |
| M/B | Abit BW7 |
| RAM | 256MB SDRAM PC-133 DIMM1 |
| NIC | Intel PRO/1000 T |
| OS HD | Seagate 40G 7200RPM 2MB Catch |

| Client 5 |  |
| --- | --- |
| CPU | Intel P4-1.5G |
| M/B | Abit TH7-II |
| RAM | 256MB RDRAM SIMM1 SIMM2 |
| NIC | Intel PRO/1000 T |
| OS HD | Seagate 20G 7200RPM 2MB Catch |

Switch Configuration

| Vendor | 3Com |
| --- | --- |
| Model | SuperStack 3 Baseline Gigabit Switch |
| Media | Copper Gigabit Connections (Cat 5E) |

Software Configuration

| OS | Windows 2000 Advance Server/SP2 |
| --- | --- |
| Test Tool | Intel Iometer 1999.10.20 |

# Copyright

©2002 Akom Technology Corp. All rights reserved. No part of this document may be reproduced, stored in a retrieval system, or transmitted in any form or by any means, electronics, mechanical, photocopying, recording, or otherwise, without the prior written consent of Akom Technology Corp, 5F, No. 323, Yangguang St., Neihu Chiu, Taipei 114, Taiwan R.O.C.

All trademarks are owned by their respective owners.

# Disclaimer

The material in this document is for information only and is subject to change without notice. Akom assumes no liability resulting from errors or omissions in this document, or from the use of the information contained herein.